Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DESIGN OF MECHANICAL TRANSMISSION SYSTEMS     Time : 3 hours

Subject Code: 	ME231			         						                Maximum Marks: 100

#### Use of approved data hand books are permitted

Assume missing data if any suitably.

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Belt drives are ____________drives.

2.	To double the life of a ball bearing, it should be loaded with _________times the given load.

3.	__________profile spur gear teeth avoid interference.

4.	Double helical gears are also called as __________.

5.	Bevel gears are made with _________to avoid interference.

6.	_________gear drive has reversible characteristic and offers self locking facility.

7.	In a belt drive, using ___________the velocity ratio may be varied without stopping the drive.

8.	The four general types of gear boxes are ___________.

9.	___________mechanism is used to produce intermittent circular motion from an oscillating or reciprocating member.

10.	Tangent cams are generally used in ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	A 75mm long full journal bearing of diameter 75mm supports a load of 10 kN. The speed of the journal is 1200 rpm. The absolute viscosity of the oil is 10 x 10 -3 Pas and diametral clearance ratio is 0.001. Determine the co-efficient of friction using Petroff’s equation.

12.	In a ‘Block and Tackle’ mechanism, four sheaves are at the top block and four at the bottom block. The pulley co-efficient of the unit is 1.12, calculate its efficiency to raise and lower the load.

13.	A pair of level gears transmitting 7.5 kw power at 300 rpm of pinion. The pressure angle is 20. The pitch diameters of pinion and gear at their large ends are 150mm and 200mm respectively. The Facewidth of the gear is 40mm. Determine the components of the resultant gear tooth force.

14.	Determine the resulting speeds of a six speed gear box of a lathe, whose speed range is 50 to 1600 rpm.

15.	Derive the condition for maximum velocity of the reciprocating roller follower of a tangent cam when the roller has contact with its straight flanks.

PART – C  (5 x 15 = 75 MARKS)

16.	A lightly loaded journal bearing has a load of 1 kN. The oil used is SAE 60 and mean effective temperature of operation is 40C. The journal has a diameter of 50mm and the bearing has a diameter of 50.5mm. The speed of journal is 15000 rpm. The Ld ratio is limited to 1.2. Determine the co-efficient of friction and power loss in friction.

(OR)

[P.T.O]

17.	Select a V.Belt drive to transmit 10 kw of power from a pulley of 200mm diameter mounted on an electric motor running at 720 rpm to another pulley mounted on compressor running at 200 rpm. The service is heavy duty varying form 10 hours to 14 hours per day and center distance between centres of pulleys is 600 rpm.

18.	Select a ‘Roller chain drive’ to transmit power of 10 kw from a shaft rotating at 750 rpm to another shaft to be run at 450 rpm. The distance between the shaft centres could be taken as 35 pitches.

(OR)

19.	Two spur gears are to be used for a ‘Rock crusher drive’ and are to be of minimum size. The gears are to be designed for the following requirements. Power to be transmitted is 18 kw, speed of pinion 1200 rpm. Velocity ratio 3.5 :1, tooth profile 20 stub involute. Determine the module and face width for strength requirements only.

20.	A pair of Bevel gear wheels with 20pressure angle consists of 20 teeth pinion meshing with 30 teeth gear. The module is 4mm while the face width is 20mm. The surface hardness of both pinion and gear is 400 B.H.N. The pinion rotates at 500 rpm and receives power from an electric motor. The starting torque of the motor is 150 percent of the rated torque. Determine the safe power that can be transmitted considering the dynamic load, wear strength and endurance strength. The allowable bending stress may be taken as 240 MPa.

(OR)

21.	Complete the design and determine the input capacity of a worm gear speed reducer unit which consists of hardened steel worm and a phosphor bronze gear having 20 stub involute teeth. The centre distance is to be 200mm and transmission ratio is 10 and the worm speed is 2000 rpm.

22.	Design a two speed spur gear box. The unit has to transmit 30 kw power. The driving shaft runs at 1000 rpm and the driven shaft is to have two alternate speeds of 350 rpm and 260 rpm. (The actual speeds obtained to be within  1.5% of specific speeds). Sliding gears (not constant mesh) are to be used. Select Ni – Cr steel of 950 MPa tenacity for gears. The diameters of both driving and driven shafts should not exceed 55mm at bearings. Design any one pair of gears and the intermediate shaft. Gear box casting need not be detailed.

(OR)

23.	Design an automotive constant mesh type gear box for three forward speeds and a reverse one. The top gear a unity and bottom and reverse gear ratios are of approximately 3.3 :1. The approximate center distance between the shafts is to be 110mm. As an inevitable case, gear teeth of module 3.25 are to be employed.

24.	Find the resulting various stresses in screw and nut of the lead screw of a lathe having trapezoidal threads. For movement of the tool carriage, the screw has to exert an axial force of 15 kN. Consider the collar thrust. The lead screw is 1.25 m long. Co-efficients of friction at the collar and nut are 0.11 and 0.14 respectively. It has 10 threads in contact and di = 46mm, dm =50.5mm, d0 = 55m, dc = 57mm and p = 9mm. The yield stress in shear = 300 MPa.

(OR)

25.	The nominal diameter of triple start Square Screw is 50mm, while the pitch is 8mm. It is used with a collar having outer diameter of 100mm and inner diameter of 65mm. The co-efficient of friction at the thread surface as well as at the collar surface can be taken as 0.15. The screw is used to raise a load of 15 kN. Using the uniform wear threay for collar friction, calculate:

a.	Torque required to raise the load.

b.	Torque required to lower the load and

c.	The force required to raise the load, if applied at a radius of 500 mm.