Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FLUID POWER CONTROL ENGINEERING		              Time : 3 hours

Subject Code: 	ME237			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define pump

2.	What do you mean by vane?

3.	Write the types of valves.

4.	What is the use of pipeline?

5.	What is hydraulic control?

6.	Valve controls the ________.

7.	Dual cylinder has ________ cylinders.

8.	Lubricants are used in _________

9.	Write any two properties of fluid.

10.	Write any two types of pumps.

PART – B  (5 x 3 = 15 MARKS)

11.	Define radial pumps

12.	Explain relief valves.

13.	Write the uses of pneumatic power.

14.	Explain electro  pneumatic  circuit.

15.	What are synchronizing circuits?

PART – C  (5 x 15 = 75 MARKS)

16.	Write in detail the advantages of fluid power systems.

(OR)

17.	Explain with neat sketch external gear pump.

18.	Explain pressure relief valve (spring loaded type) with neat sketch.

(OR)

19.	A press with the tool weighing 5KN is used for forming. The force required for pressing is  100kN and counterbalance valve is used to counter act the weight of the tools. The cylinder with the piston diameter 80mm and rod diameter 60mm is used. Calculate the pressure to achieve 100kN pressing force.

20.	Explain surface filter, its advantages and disadvantages.

(OR)

21.	Compare the proportional and servo valves in detail.

22.	Write the basic types of accumulators used in hydraulic system. Explain any one with neat 	sketch.

(OR)

23.	a. What are servo valves, where they are used.    b. Explain mechanical hydraulic servo valve.

24.	Write in detail the trouble shooting of hydraulic systems.

(OR)

25.	Explain with block diagram the components in a PLC and give their functions.