Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FLUID MECHANICS								               Time : 3 hours

Subject Code: 	ME251			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Pascal’s Law?

2.	What is the use of Pitot tube?

3.	What is Velocity potential?

4.	What is Stream line?

5.	What is Specific speed of pump?

6.	What is NPSH?

7.	What is Mach number?

8.	What is Stagnation pressure?

9.	What is Supersonic flows?

10.	What is Jet propulsion?

PART – B  (5 x 3 = 15 MARKS)

11.	Give different Pressure gauge instruments

12.	Write major and minor energy losses in pipe

13.	Why is priming required in pumps

14.	Write an expression for momentum equation for compressible fluid for isothermal process

15.	What is Static pressure ratio

PART – C  (5 x 15 = 75 MARKS)

16.	State Bernoulli’s theorem for steady flow of an incompressible fluid. Derive an expression for Bernoulli’s theorem from first principle and state the assumptions made for such a derivation

(OR)

17.	a.	How would you determine the horizontal and vertical components of the resultant pressure

on a sub-merged curved surface.											(8)

b.	Explain the procedure of finding Hydrostatic forces on curved surfaces.			(7)

18.	A fluid flow field is given by V = x2yi + y2zj – (2xyz + yz2)k. prove that it is a case of possible steady incompressible fluid flow. Calculate the velocity and acceleration at the

point ( 2, 1, 3 )

(OR)

19.	A siphon of diameter 200mm connects two reservoirs whose water surface level differs by 40m. The total length of the pipe is 8000m. The pipe crosses a ridge. The summit of ridge is 8 m above the level of water in the upper reservoir. Determine the minimum depth of the pipe below the summit of the ridge, if the absolute pressure head at the summit of siphon is not to fall below 3m of water. Take f = 0.006 and atmospheric pressure head = 10.3m of water. The length of siphon from the upper reservoir to the summit is 500m. Find discharge also

[P.T.O]

20.	The internal and external diameter of an impeller of a centrifugal pump which is running at 1000rpm are 200mm and 400mm respectively. The discharge through pump is 0.04 m3/s and velocity of flow is constant and equal to 2m/s. the diameters of the suction and delivery pipes are 150mm and 100mm respectively and suction and delivery heads are 6m (abs) and 30m (abs) of water respectively. If the outlet vane angle is 450  and power required to drive the pump is 16.186Kw.  Determine i) Vane angle of the impeller at inlet     ii) the overall efficiency of the pump  iii)  Manometric efficiency of the pump

(OR)

21.	a.	With a neat diagram explain the working of a Reciprocating pump.						(8)

b.	Derive the expressions for Discharge through a reciprocating pump and work done by the

reciprocating pump.              														(7)

22.	Derive the expression for velocity of sound wave in fluid.

(OR)

23.	a.	Derive the expression for mass rate of flow of compressible fluid through

Venturi meter                                                                  								(8)

b.	Find the mass rate of flow of air through a venture meter having inlet diameter 300mm and

throat diameter 150mm. the pressure at the inlet of Venturimeter is 1.4kgf/cm2 absolute and temperature of air at the inlet 150c. the pressure at the throat is 1.3kgf/cm2 absolute. Take           R = 287J/kg.K and k= 1.4                                                   								(7)

24.	Explain in detail flow with normal shock

(OR)

25.	Write a short note on:

a.	Thrust equations 	 b.	liquid propellant   c.	Turbo Jet components