Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICS OF MACHINES - I					              Time : 3 hours

Subject Code: 	ME254			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a kinematic pair, when the elements have surface contact while in motion, it is a _________________ pair.

2.	The magnitude of linear velocity of a point B on a link AB relative to point A is ___________________.

3.	The semi-cone angle in a cone clutch is always kept greater than the angle of static friction to avoid _______________.

4.	The rotor of a ship rotates in clockwise direction when viewed from the stern and the ship takes a left turn. The effect of the gyroscopic couple acting on it will be to ___________________________________________.

5.	Angle through which the cam turns while the follower remains stationary at the highest or the lowest position is called as ___________________.

6.	In ____________ follower, the line of movement of the follower passes through the centre of rotation of the cam.

7.	The radial distance between the pitch circle of a gear and its addendum circle is called as __________________.

8.	A gear box has an input speed of 1500 rpm clockwise and an output speed of 300rpm anticlockwise. The input power is 20kW and the efficiency is 70%. Determine the input torque of the gear box.

9.	When the sleeve of a Porter governor moves upwards, the governor speed ___________.

10.	When the speed of the engine fluctuates continuously above and below the mean speed, then the governor is said to be _________________.

PART – B  (5 x 3 = 15 MARKS)

11. 	In a four bar chain ABCD, AD is fixed and is 150mm long. The crank AB is 40mm long and rotates at 120 rpm clockwise, while the link CD = 80 mm oscillates about D. BC and AD are of equal length. Find the angular velocity of the link CD when angle BAD = 600.

12.	Determine the maximum pressure in a plate clutch when the axial force is 4kN. The inside radius of the contact surface is 50mm and the outside radius is 100mm. Assume uniform wear.

13.	Why a roller follower is preferred to that of a knife-edged follower?

14.	The following data relate to a pair of 200 involute gears in mesh:

Module = 6mm, number of teeth on pinion = 17, number of teeth on gear = 49, addenda on pinion and gear wheel = 1 module. Find the radius of addendum circle of gear.

15.	In a spring controlled gravity governor, the mass of each ball is 1.6kg. Distance of fulcrum from the axis of rotation is 60mm. The bell crank lever has a vertical arm 120mm long and a horizontal arm 50mm long. The mass of the sleeve is 6.5kg. The sleeve begins to rise at 200rpm and the rise of sleeve for 5% increase is 9mm. Determine the initial thrust in the spring.

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	The crank of a slider crank mechanism rotates counterclockwise at a constant speed of 20 rad/s. The crank is 480mm and the connecting rod is 1600mm long. Determine:

i	the acceleration of the slider

ii	angular velocity and angular acceleration of the connecting rod, at a crank angle of 600 from the inner dead centre position.

(OR)

17.	PQRS is a four bar chain with link PS is fixed. The lengths of the links are PQ =62.5 mm; QR=175mm; RS=112.5mm and PS=200mm. The crank PQ rotates at 10 rad/s clockwise. Draw the velocity and acceleration diagram when angle QPS=600 and Q and R lie on the same side of PS. Find the angular velocity and angular acceleration of links QR and RS.

18.	 A cone clutch with a semi-cone angle of 150 transmits 10Kw at 600rpm. The normal pressure intensity between the surfaces in contact is not to exceed 100kN/m2. The width of the friction surfaces is half of the mean diameter. Assume coefficient of friction = 0.25. Determine:

i	the outer and inner diameter of the plate

ii	width of the cone face

iii	the axial force to engage the clutch

(OR)

19.	Each wheel of a motor cycle is of 600 mm diameter and has a moment of inertia of 1.2 kgm2. The total mass of the motorcycle and the rider is 180kg and the combined centre of mass is 580mm above the ground level when the motor cycle is upright. The moment of inertia of the rotating parts of the engine is 0.2 kgm2. The engine speed is 5 times the speed of the wheels and is in the same sense. Determine the angle of heel necessary when the motorcycle takes a turn of 35m radius at a speed of 54kmph.

20.	A cam with a minimum radius of 25mm rotating clockwise at a uniform speed is to be designed to give a roller follower, at the end of a valve rod, motion described below:

i	to raise the valve through 50mm during 1200 rotation of the cam

ii	to keep the valve fully raised through next 300

iii	to lower the valve during next 600 and

iv	to keep the valve closed during rest of the revolution

The diameter of the roller is 20mm and the diameter of the cam shaft is 25mm. Draw the profile of the cam when the line of the stroke is offset 15mm from the axis of the cam shaft.

(OR)

21.	A cam drives a flat reciprocating follower in the following manner: During first 1200 rotation of the cam, follower moves outwards through a distance of 20mm with simple harmonic motion. The follower dwells during next 300 of cam rotation. During next 1200 of cam rotation, the follower moves inwards with simple harmonic motion. The follower dwells for the next 900 of cam rotation. The minimum radius of the cam is 25mm. Draw the profile of the cam.

22.	An epicyclic train of gears is arranged as shown in Fig. 1 below. How many revolutions does the arm A, to which the pinions B are attached, make

i	when D makes one revolution clockwise and C makes half a revolution anti-clockwise

ii	when D makes one revolution clockwise and C is stationary?

Fig.1. Epicyclic train of gears

(OR)

23.	Two involute gears of 200 pressure angle are in mesh. The number of teeth on pinion is 20 and the gear ratio is 2. If the pitch expressed in module is 5mm and the pitch line speed is 1.2 m/s, assuming addendum as standard and equal to one module, find:

i	the length of arc of contact

ii	angle turned through by pinion when one pair of teeth is in mesh

iii	the maximum velocity of sliding.

24.	Each arm of a Porter governor is 250mm long. The upper and lower arms are pivoted to links of 40mm and 50mm respectively from the axis of rotation. Each ball has a mass of 5kg and the sleeve mass is 50kg. The force of friction on the sleeve of the mechanism is 40N. Determine the range of speed of the governor for the extreme radii of rotation of 125mm and 150mm.

(OR)

25.	A governor of the Proell type has each arm 250mm long. The pivots of the upper arm and lower arms are 25mm from the axis. The central load acting on the sleeve has a mass of 25kg and the each rotating ball has a mass of 3.2kg. When the governor sleeve is in mid-position, the extension link of the lower arm is vertical and the radius of path of rotation of the masses is 175mm. If the governor speed is 160rpm when in mid-position, find the length of the extension link and the tension in the upper arm.