Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	INSTRUMENTATION AND CONTROL SYSTEMS	              Time : 3 hours

Subject Code: 	ME256			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example for integrating instrument.

2.	In a generalized measurement system, the function of the signal manipulating element is ____________.

3.	What is LVDT?

4.	What is a load cell?

5.	Write the expression for gauge factor of a strain gauge.

6.	Piezoelectric crystals are used for measurement of ______________ changes.

7.	State Mason’s gain formula.

8.	What is the effect of integral control?

9.	What is an order of a system?

10.	Draw the response of a first order system when subjected to unit step input.

PART – B  (5 x 3 = 15 MARKS)

11.	What is instrumental Errors? How it can be avoided.

12.	What is thermistor? State it’s principle of operation.

13.	Define the terms absolute humidity, relative humidity and dew point.

14.	List the advantages of closed-loop system over open-loop system.

15.	State Nyquist stability criterion.

PART – C  (5 x 15 = 75 MARKS)

16.	Name the various forces acting on an indicating instrument and explain each with neat diagram.

(OR)

17.	a.	Explain the various elements of a generalized measurement system with a  neat block

diagram. 							     										(10)

b.	Discuss about Gross errors and also mention the methods through which it can be eliminated.							       										(5)

18.	a.	With a neat sketch explain the operation of electro magnetic flow-meter. 			(8)

b.	Explain how a strain gauge is used to measure torque. 		       					(7)

(OR)

19.	Discuss the pressure measurement technique using bourdon tube with LVDT.

20.	With a neat sketch explain the operation of the absorption type hydrometer.

(OR)

21.	What is piezoelectric effect? Explain method of measuring acceleration by using piezoelectric transducer.

[P.T.O]

22.	For the block diagram show in Fig.1 construct (i) equivalent signal flow graph for the block diagram (ii) obtain C/R by the application of Mason’s   formula.

G1

G2

G3

R

+

+

+

+

H1

H2

H3

C

-

-

-

x1

x2

x3

x4

x5

x6

Fig. 1

(OR)

23.	Use Mason’s gain formula for determining the overall transfer function of   the system show in

Fig.2

R(s)

C(s)

G1

G2

G3

G4

G5

G6

H1

1

G7

H2

H3

Fig. 2

24.	Obtain the unit impulse response and the unit step response of a unity   feedback system whose open-loop transducer function is G (S) =2s+1s2.

(OR)

25.	a.	Using Routh criterion determine the range of values of K for stability for   the following

system C(S)R(S) = KS(S2+S+1)(S+4)+K.		     								(10)

b.	Draw the approximate polar plot for the following function.                     			(5)

i.	GH(S) = 	1(1+T1S)(1+T2S)	ii.	GH(S) = 	1S2(1+TS)