Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DESIGN OF MACHINE ELEMENTS				              Time : 3 hours

Subject Code: 	ME257			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When the material is loaded within elastic limit, then the stress is ________to strain.

2.	Rankine’s theory is used for __________materials.

3.	The stress which vary from a minimum value to a maximum value of the same nature is called ____________.

4.	When a material is subjected to fatigue loading, the ratio of the endurance limit to the ultimate tensile strength is __________.

5.	For the design of axles, the twisting moment only is considered. (True / False)

6.	When a pulley or other mating piece is required to slide along the shaft, a ________sunk key is used.

7.	The centre to centre distance between two consecutive rivets in a row, is called as _______.

8.	An oral type flange is fastened by means of two bolts. (True or False).

9.	The length of the piston usually varies between _________where D is the diameter of the piston.

10.	Due to the centrifugal force acting on the rim, the flywheel arms will be subjected to ______stress.

PART – B  (5 x 3 = 15 MARKS)

11.	Which is called as stress concentration?

12.	Define endurance limit.

13.	What is a key?

14.	Name the three commonly used cotter joints.

15.	Define the terms ‘ coefficient of fluctuation of speed’ and ‘coefficient of steadiness’.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With the help of a block diagram, explain the general procedure in machine design.		(7)

b.	A steel bracket is subjected to a load of 4.5kN as shown in figure 1. Determine the required

thickness of the section in order to limit the tensile stress to 70 MPa.

50mm

4.5 kN

Fig.1

100mm

t

50mm

(OR)

17.	At a critical point on a machine part, the principal stresses are 50 N/mm2 and 40 N/mm2. Find the safe tensile stress in the following:

Ductile material, stresses of same nature, according to any three theories of failure.

[P.T.O]

18.	A steel machine member is subjected to axial loading varying from 300 N/mm2 tension to

200 N/mm2 compression. The tensile yield strength of the material is 650 N/mm2 and the flexural endurance limit for fully reversed harmonic loading is 45 N/mm2, including the effect of the surface finish of the machine member. Determine the permissible working stress for a design factor of 1.4.

(OR)

19.	A locomotive semi-elliptical laminated spring has an overall length of 1m and sustains a load of 75 kN at its centre. The spring has 3 full length leaves and 15 graduated leaves with a central band of 100m width. All the leaves are to be stressed to 410 MPa, when fully loaded. The ratio of the total spring depth to width is 2. E = 210 kN/mm2. Determine.

a.	The thickness and width of the leaves.

b.	The initial gap that should be provided between the full length and graduated leaves before

the band load is applied

c.	The load exerted on the band after the spring is assembled.

20.	Determine the diameter of hollow shaft having inside diameter 0.5 times the outside diameter. The permissible shear stress is limited to 200 MPa. The shaft carries a 900mm diameter cast iron pulley. The pulley is driven by another pulley mounted on the shaft placed below it. The belt ends are parallel and vertical. The ratio of tensions in the belt is 3. The pulley on the hollow shaft weights 800N and over hangs the nearest bearing by 250mm. The pulley is to transmit 35 kW and 400 rpm.

(OR)

21.	Design a muff coupling which is used to connect two steel shafts transmitting 25 kW at 300 rpm. The allowable shear and crushing stresses for the material of the shafts and key may be taken as 40 MPa and 70 MPa respectively. The allowable shear stress for the material of the muff may be assumed as 15 MPa.

22.	Two lengths of mild steel flat tie bars 200mm x 15mm are to be connected by a double riveted double cover butt joint, using 20mm diameter rivets. Design the joint, if the allowable stresses are 110 MPa in tenstion, 80 MPa in shear and 200 MPa in crushing. Sketch the arrangement.

(OR)

23.	Design the flanged joint for a cast iron pipe 200mm diameter to carry a fluid pressure of

0.8 N/mm2.

24.	Design a plain carbon steel centre crankshaft for a single acting four stroke, single cylinder engine for the following data:

Piston diameter = 250mm		:		Stroke 	= 	400mm:

Maximum combustion pressure = 2.5 N/mm2

Weight of the flywheel = 18 kN

Total belt pull = 4N

Length of connecting rod = 900mm.

When the crank has turned through 30º from top dead centre, the pressure on the piston is

1 N/mm2 and the torque on the crank is maximum. Any other data required for the design may be assured.

(OR)

25.	A multi-cylinder engine is to run at a speed of 500 rpm. The crank effort diagram is drawn to a scale of 1mm = 2000 N – m and 1mm = 3 º crank angle. The areas above and below the mean torque line are in sq. mm as below:

+ 160, 	-172,	+168,	-191,	+ 197,	-162

The speed is to be kept within  1% of the mean speed of the engine. Design a suitable rim type C.I fly wheel for the above engine. Assume rim width as twice the thickness. The permissible stresses for the rim in tension is 6 MPa and for shaft and key in shear are 40 MPa. The allowable stress for the arm is 14 MPa.