Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	DESIGN OF HEAT EXCHANGERS AND PRESSURE VESSELS

FOR FOOD PROCESSING							              Time : 3 hours

Subject Code: 	ME261			         						                Maximum Marks: 100

Heat and mass transfer data book is permitted

Any missing data may be suitably assumed

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

a

1.	The value Uo will be__________ compared to value of Ui  in heat exchangers.

2.	NTU is defined as ________ .

3.	Capacity ratio when cold fluid is having lower heat capacity is defined in terms of terminal temperatures as_____________ .

4.	The three main flow arrangements in heat exchangers are _____________, _____________ and _____________ flow.

5.	Compact heat exchangers use _________ tube bundles.

6.	In a condenser the controlling resistance will be on the _____________ side.

7.	Air preheaters in large thermal plants use ___________________ type of heat exchangers.

8.	If heat capacities are equal for the counter flow type the effectiveness is ______.

9.	Thermodynamically the flow direction that will give minimum loss of available energy is _______________.

10.	For a condenser /evaporator the effectiveness is ________________.

PART – B  (5 x 3 = 15 MARKS)

11.	Sketch a single tube-counter flow heat exchanger.

12.	Specify the two parameters that influence the value of correction factor.

13.	What is the salient feature of a surface condenser?

14.	What amount of water is lost with induced draft cooling tower and how?

15.	What is a single effect evaporator?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive from basics the expression for effectiveness for a counter flow heat exchanger with

Ch = CC  or (Cmin / Cmax ) = 1.

(OR)

17.	Explain the thermal resistances encountered in heat transfer from one fluid to another. Give the necessary equations.

18.	Determine the area required for a shell and tube heat exchanger with two tube passes to cool oil at a rate of 10kg/s from 60oC to 30oC flowing in the shell using water at 20oC passing through the tubes and heated up to 26oC. The specific heat of the oil is 2200 J/kgK. The value of heat of overall heat transfer coefficient is 300 W/m2K.

(OR)

[P.T.O]

19.	The inlet and outlet temperatures of hot and cold fluids in a double pipe heat exchanger are 220oC, 100oC and 80oC and 120oC. Determine whether the exchanger is parallel flow or counter flow. Also determine the LMTD and effectiveness of the exchanger and the capacity ratio.

20.	A heat exchanger is to be designed for a capacity of 100kW. Water for air conditioning purpose is to be cooled from 15oC to 5oC in a counter flow arrangement using brine available at -20 oC. The value of overall heat transfer coefficient is 650W/m2K and may be assumed to be constant. Determine the area required if the brine outlet temperature is -10, -5 and 5oC. Comment on the results.

(OR)

21.	With the aid of sketches, explain the different types of cooling tower.

22.	An economizer in a boiler has flow of water inside the pipes and hot gases on the outside flowing across the pipes. The flow rate of gases is 2,000 tons/hr and the gases are cooled from 390o to 290o. The specific heat of the gas is 1005 J/kg K. Water is heated (under pressure 0 from 100o C to 220o C. Assuming an overall heat transfer coefficient of 35 W/m2 K, determine the area required. Assume that the air flow is mixed.

(OR)

23.	Explain in detail the different types of air heaters.

24.	Explain the horizontal and vertical types of evaporator units with sketches.

(OR)

25.	Explain in detail the criteria for selecting evaporators.