Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICAL SYSTEM FOR FOOD PROCESSING              Time : 3 hours

Subject Code: 	ME262			         						                Maximum Marks: 100

Any missing data may be suitably assumed.

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In axial piston pump, rotation is imparted to _____________.

2.	The maximum pressure (in kPa) that can be developed in a vane pump is _________.

3.	Power transmission by belt drive can be increased by increasing the coefficient of friction.

Say True/False.

4.	The size of a gear is specified by _________________.

5.	____________ is used to recover waste heat in boiler.

6.	Cochran boiler is of ____________________ type based on furnace position.

7.	A faucet is provided for _________________ in pressure type water cooler.

8.	_____________ or ______________ is used as an insulator in ice plants.

9.	____________ is the mass that is to be handled at one time.

10.	Fork lift trucks move materials on ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	When do we use multistage pumps?

12.	What are the disadvantages of flat belt drive?

13.	What is the principle of operation of a Bourdon tube?

14.	Define C.O.P.

15.	What is an automated guided vehicle?

PART – C  (5 x 15 = 75 MARKS)

16.	A centrifugal pump is to discharge 0.118 m3/s at a speed of 1450 rpm against a head of 25m. The impeller diameter is 250mm, its width at outlet is 50mm and manometric efficiency is 75%. Determine the vane angle at the outer periphery of the impeller.

(OR)

17.	Explain the working of a screw pump and an axial piston pump with the aid of neat sketches.

18.	A belt of density 1gm/ cm3 has a maximum permissible stress of 250N/mm2. Determine the maximum power that can be transmitted by a belt of 20cm x 1.2 cm if the ratio of the tight side to slack side tension is 2.

(OR)

19.	Find the maximum torque that can be safely applied to a shaft of 200mm diameter if the permissible angle of twist is 1deg. in a length of 5m and the permissible shear stress is 45N/mm2. Take rigidity modulus as 0.8 N/mm2 .

20.	Describe the constructional features and working of a Babcock and Wilcox boiler.

(OR)

21.	Explain the working of spring loaded safety valve and fusible plug.

22.	Explain, with sketches, the pressure type water cooler and remote type cooler.

(OR)

23.	With the aid of a schematic diagram, explain the working of an ice plant.

24.	Explain the basic types of material handling equipment with the aid of sketches.

(OR)

25.	Discuss in detail the conveyor operations and features.