Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	FINITE ELEMENT ANALYSIS						              Time : 3 hours

Subject Code: 	ME269			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Finite element method is a piecewise approximation technique. (True/False)

2.	Rayleigh-Ritz method can be applied to solve any differential equation. (True/False)

3.	A plane truss element has _________ degrees of freedom in global coordinate system and

____________ degrees of freedom in local coordinate system.

4.	Draw the plane beam element.

5.	What are local coordinates?

6.	What is p-method of convergence?

7.	Thermal problems are easier to formulate than solid mechanics problems. (True/False)

8.	Higher order Serendipity elements have internal nodes. (True/False)

9.	The maximum stress within the element occurs at the Gauss point (True/False).

10.	Mention two examples for plane strain approximation?

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the principle of Rayleigh-Ritz method.

12.	Write the properties of shape functions.

13. 	For the following structure (Fig. 1), explain briefly the selection of elements.

Fig. 1

14.	Write the general Helmholtz equation, and also show how this equation is modified for heat transfer, torsion and flow problems.

15.	What are the advantages of isoparametric elements?

PART – C  (5 x 15 = 75 MARKS)

16.	Find the approximate deflection of a simply supported beam under uniformly

distributed (p) load using the Galerkin method. The differential equation is given by

EId4vdx4 − p = 0, 0≤ x ≤ l

[P.T.O]

The boundary conditions to be satisfied are:

v (x=0) = 0,v(x = l) =0,d2vdx2|(x = 0) =0,andd2vdx2|(x = l) =0

Assume v(x) = C sinπxl.

(OR)

17.	a.	Explain the concept of FEM briefly and outline the procedure.					(7)

b.	Discuss the advantages and disadvantages of FEM over classical method and finite

difference method.														(8)

18.	Calculate the element strains and stresses for a stepped bar shown in Fig. 2. Take young’s modulus E = 200 GPa.

5 kN

A1 = 400 mm2

A2 = 250 mm2

200 mm

250 mm

Fig. 2

(OR)

19.	For the beam shown in Fig. 3 calculate the global stiffness matrix and load vector. Assume

E = 70 GPa, I =  and P = 5 kN

Fig. 3

20.	a.	State and explain the convergence requirements of polynomial shape functions. 		(7)

b.	Explain briefly the different methods of measuring the element distortion. 			(8)

(OR)

21.	Derive the shape function equations for a CST element in local coordinates. Also plot the variation of shape functions over the element face.

22.	What are the advantages of higher order elements? Also derive the shape function equation for a second-order Lagrange element.

(OR)

23.	Write short notes on the following:												  (8+7)

a.	Lagrange and Serendipity elements		b.	Gauss quadrature method of integration

24.	Calculate the element stiffness matrix for the plane stress element shown in Fig. 4. Assume plate

thickness = 5 mm, young’s modulus = 150 × 103 MPa and Poisson’s ratio = 0.25.

3 (1, 3)

1 (0,0)

2 (2,0)

Fig. 4

(OR)

25.	A wall of an industrial oven consists of three different materials. The first layer is composed of

5 cm of insulating cement with a clay binder that has a thermal conductivity of 0.08 W/m K. The second layer is made from 15 cm of asbestos board with a thermal conductivity of 0.074 W/m K. The exterior consists of 10 cm common brick with a thermal conductivity of 0.72 W/m K. The inside wall temperature of the oven is 200°C, and the outside air is 30°C with a convection coefficient of 40 W/m2 K. Determine the temperature distribution along the composite wall.