Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	MECHANICAL VIBRATIONS						              Time : 3 hours

Subject Code: 	ME270			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is natural frequency of a vibrating System?

2.	Define the degree of freedom of a vibrating system?

3.	Why does the amplitude of free Vibration gradually diminish in practical systems?

4.	State the principle of virtual displacement.

5.	What are normal mode vibrations?

6.	What is flexibility matrix?

7.	State the boundary conditions of a beam in which one end is fixed and other end is free.

8.	Rayleigh’s is method can be applied to find the ________natural frequency of continuous system.

9.	What are the two type of real – time analysis procedures? Give their best suited analysis.

10.	State the main advantage of a Stroboscope.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the concept of discrete and continuous system.]

12.	A body of meas 0.25 kg is suspended by a spring having a stiffness of 0.1533 N/mm. Determine its natural frequency in cycle per second. Determine its statistical deflection.

13.	What are the factors to be considered for choosing the type of vibration measuring instruments in a vibration test.

14.	State some methods to control vibrations.

15.	What are the hardwares required for measurement of vibration in an experimental modal analysis?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive an expression for the equivalent spring constant of springs connected in parallel.

(OR)

17.	Derive an expression for damping constant of the dash pot shown in the figure (1).

18.	Derive the equation of motion of a spring – mass system in vertical position.

(OR)

19.	Use the principle of conservation of energy to derive the equation of motion.

20.	Estimate the fundamental frequency of vibration of the system shown in figure (2).

(OR)

21.	Derive the orthogonality principle for normal functions for the longitudinal vibration of bars.

22.	Derive the equation of motion and solution for the longitudinal vibration of a rod of length1 with varying cross section.

(OR)

23.	Determine the natural frequencies of vibration of a uniform rod clamped at one end and free at the other.

24.	Explain the principle of operation of an Linear Variable Differential Transformer Transducer with a schematic diagram.

(OR)

25.With a neat diagram explain how vibration could be measured with the help of an piezo electric accelerometer.