Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED STRENGTH OF MATERIALS			              Time : 3 hours

Subject Code: 	ME353			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Determine the shear stress distribution for a circular open section under bending caused by a shear force. Locate the shear centre. (Fig. 1)										(10)

Y

F



Fig.1

R

0

2R

b.	Describe briefly about shear centre with suitable example.							(10)

2.	a.	Derive the equation of equilibrium in cylindrical co-ordinates.						(12)

b.	Explain the state of pure shear.													(8)

(OR)

3.	a.	Derive the equation of compatibility.											(12)

b.	Derive the stress-strain relations for Isotropic materials.							(8)

4.	Describe briefly about Theories of failure.

(OR)

5.	Determine the diameter d of a circular shaft subjected to a bending moment M and a Torque T according to the several theories of failure. Use a factor of safety N.

6.	a.	Explain briefly the first theorem of Castigliano.									(12)

b.	Determine the deflection at end A of the cantilever beam, shown in Fig. 2.			(8)

x

Q								   P

Fig.2

L

W

(OR)

[P.T.O]

7.	a.	Discuss briefly the second theorem of Castigliano.								(12)

b.	A cantilever is supported at the free end by an elastic spring of spring constant K. Determine

the reaction at A (Fig. 3). The cantilever beam is uniformly loaded. The intensity of loading is 																			(8)



Fig.3

A

(OR)

8.	Discuss briefly the stresses in composite tubes.

(OR)

9.	Derive the displacement, stress equations for thick – walled cylinder subjected to Internal and External pressures. The problem treated as plane stress case.