Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRONICS - I									               Time : 3 hours

Subject Code: 	MT103			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define free electron.

2.	What do you mean by rectifier?

3.	Mention the different operating regions of transistor.

4.	Which configuration provides better current gain?

5.	When no gate voltage, a --------------- behaves as an ordinary PN junction diode.

6.	Why is the input impedance of a MOSFET higher than that of FET?

7.	Define DC load line.

8.	What is drain resistance of FET?

9.	State the advantage of cascode amplifier.

10.	Define CMRR.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the basic action of clipper circuits.

12.	Define stability factor.

13.	Compare JFET with MOSFET.

14.	What is Miller effect?

15.	Write a note on effects of coupling capacitors in transistor amplifiers.

PART – C  (5 x 15 = 75 MARKS)

16.	With the help of energy band diagram, distinguish between conductor, insulator, and semi conductor.

(OR)

17.	What are the classifications of clamper circuit? Explain the operation of clampers with necessary diagrams.

18.	Draw and explain the input and output characteristics of a transistor in CB configuration. And also indicate a cut off, saturation and active regions.

(OR)

19.	Mention the requirements of a biasing circuit and discuss any one technique for biasing of BJT.

20.	Illustrate the drain and transfer characteristics of n-channel enhancement type MOSFET.

(OR)

21.	Describe the methods of biasing JFET with the help of circuit diagram and expressions.

22.	Write a short note on “Transistor as an amplifier”. Draw and explain the operation of CE, CC, CB amplifier circuits with the use of circuit diagrams.

(OR)

23.	Discuss about the circuit diagram and equivalent model of source follower with a neat sketch.

24.	With a neat circuit diagram, explain the operation of cascode amplifier.

(OR)

25.	Explain the differential mode and common mode operation of differential amplifier using circuit diagrams.