Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	COMPUTER INTEGRATED MANUFACTURING SYSTEMS		Time : 3 hours

Subject Code: 	PE329			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a. 	What are the types of production systems? Explain with examples. 						(7)

b. 	What are the basic functions required to convert the raw materials into

finished product?																(5)

c. 	Define the following terms, related to analysis of manufacturing operations.				(8)

i. 	Manufacturing Lead Time (MLT)		ii. 	Plant Capacity (PC)

iii. 	Utilization and availability			iv. Work-in-process (WIP)

2.	a.	What is a part family? Explain any one type of classification and coding scheme with its

structure.																		(8)

b. 	Explain the composite part concept in machine cell.									(6)

c. 	What are the types of machine cell design? Explain.									(6)

(OR)

3.	a.	What are the benefits of classification and coding systems? Explain how these systems are

helpful in making a process plan?													(6)

b.	Explain the steps involved in retrieval and generative type Computer Aided Process Planning (CAPP) systems with examples.													(14)

4.	a.	Explain the principal functions and activities must be performed in a production planning

system with a block diagram. 													(10)

b. 	What are the inputs to the MRP system? Sketch the structure of a MRP system and explain

how it works? 																(10)

(OR)

5	a.	What are the three phases in shop floor control? Explain in detail.						(12)

b. 	What are the technologies available for automatic identification? Explain any two

of them. 																		(8)

6	a. 	What is meant by Direct Digital Control (DDC)? What are the components of DDC? Sketch

the arrangement of these components for process control. Explain the control actions in DDC with an example.																		(12)

b.	Explain how regulatory and feedforward control objectives established in supervisory

computer control. 																(8)

(OR)

7	a. 	Briefly explain the following methods of contact inspection 							(12)

i. 	Coordinate Measuring Machines

ii.	Flexible inspection systems

b. 	Define Machine vision. Explain how machine vision technique is used as a non contact inspection technique? 																	(8)

8	a. 	What is an FMS? What are its components?											(4)

b. 	What are the functions material handling systems in FMS?								(4)

c. 	Explain the types of FMS layout configurations with neat sketch.						(12)

(OR)

9	a. 	What are the functions performed by the FMS computer control system? Explain.			(10)

b. 	What is meant by rapid prototyping? How it is different form conventional prototype

making process? Explain the steps involved in rapid prototyping process. 				(8)

c. 	State any four types of rapid prototyping techniques.									(2)