Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ELECTRONICS									              Time : 3 hours

Subject Code: 	PH322			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	With suitable diagram explain the construction and working of a n- channel JFET.		(10)

b.	Apply proper biasing between drain and source and sketch the depletion region

for Vgs = OV.																(5)

c.	Draw the Transfer characteristics directly from the drain characteristics and explain the

various regions of operation.													(5)

2.	Describe the five steps involved in fabricating a monolithic integrated circuit with suitable

3

4

diagram for the equivalent circuit given blow.

2

1

5

(OR)

3.	Explain in detail the following processes involved in Fabricating a monolithic IC.(5+5+5+5)

i.	Epitaxial Growth			ii.	Masking and Etching

iii.	Diffusion of Impurities		iv.	Metalisation

4.	a.	Give the schematic representation of Basic operational amplifier and explain its

characteristics.																(10)

b.	With suitable summing amplifier diagram and virtual ground equivalent circuit calculate the

output voltage of an op-amp for the voltages and resistors as

V1 = 1v,			V2 = 2v, 	V3 = 3v,		R1 = 500 k,		R2 = 1 M,	R3 = M

The voltage shunt feed back resistance Rf = 1 M									(10)

(OR)

5.	a.	With the virtual ground equivalent circuit explain how the op-amp will act as

an Integrator. 																	(10)

b.	Explain how op-amp can be modified as monostable multivibrator.						(10)

6.	Describe Internal Architecture of 8086 microprocessor and explain the functional parts of the

Execution unit.

(OR)

7.	a.	Explain the 9 addressing modes of 8086 microprocessor.								(10)

b.	Describe the various instructions classified into six functional groups.					(10)

8.	a.	Explain the four types shift registers and draw a block diagram for each.					(10)

b.	Describe the construction and working of a ripple counter using a clocked JK

flip-flops.																	(10)

(OR)

9.	a.	Explain how synchronous counter is superior in terms of number of flip flops and time delay.

Explain it with applying clock pulse by synchronous counter.							(10)

b.	Draw the logic diagram of 2176 EPROM memory and explain the salient features.			(10)