Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – July 2008

Subject Title: 	DESIGN OF MECHANICAL TRANSMISSION SYSTEMS     Time : 3 hours

Subject Code: 	ME231				         						          Maximum Marks: 60

#### Suitably assume the missing data if any

#### Use of Data Hand Book is permitted

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A bearing in which the amount of heat generated is almost equal to the heat dissipated is called 	a ________.

2.	In an open belt drive, generally the belt tensions are to the calculated using the angle of wrap of 	___________ pulley.

3.	To design helical gears on the lines of spur gear design, _________pitch value should be 	considered.

4.	To avoid ‘interference’ in a gear drive, ______shape teeth are to be adopted.

5.	In bevel gear design, always the ________end tooth data should be considered.

6.	A speed reduction as high as 100 : 1 can be obtained using ______gear drive.

7.	_________pulleys are used to vary the velocity ratio by simply shifting the belt from one pair to 	the another one.

8.	Classification of “Ray diagrams’ are based on ___________.

9.	Maximum efficiency of power screws may be achieved by adopting the helix angles between 	_________.

10.	A ‘Ratchet and Pawl’ mechanism is an example for __________motion.

PART – B  (5 x 2 = 10 MARKS)

11.	Journal bearings and ball bearings based on load.

12.	Usage of sprockets and grooved pulleys during power transmission.

13.	Applications of worm and worm gear drive as speed increaser and speed decreaser.

14.	Position of output shaft and counter shaft with respect to the driver shaft.

15.	Co-efficient of thread friction and co-efficient of Collar friction in power screws.

PART – C  (5 x 8 = 40 MARKS)

16.	Find the ‘Frictional power loss’ for a petroff’s bearing 80mm in diameter and 120mm long. The 	radial clearance is 0.05mm and the speed of the journal is 900 rpm. The oil used is SAE 10 and 	its operating temperature is 80C.

(OR)

17.	A Compressor requiring 90 kw is to run at 250 rpm. The drive is by V-Belt from an electric 	motor running at 750 rpm. The diameter of the pulley on the Compressor shaft is 1m, while the 	centre distance between the pulleys is limited to 1.75m. The belt speed should not exceed          	1600 m/min. Determine the number of V-Belts required to transmit the power, if each belt has a 	cross sectional area of 375 mm2 and density of 1mg/m3 and has an allowable stress of 2.5 	M.Pa. 	The groove angle of the pulley is 35 and co-efficient of friction between the belt and the pulley 	is 0.25.

[P.T.O]

18.	Two spur gears are to be used intermittently for a ‘Rock crusher drive’. The gears are to be 	designed for the following requirements: No. of the teeth on pinion = 16. power to be 	transmitted 	= 20 kw, speed of pinion = 100 rpm. Velocity ratio = 3.5 to 1, 						Tooth profile = 20 stub.

Static stress for pinion material = 100 M.Pa.

Static stress for gear material = 70 M.Pa.

Determine the necessary facewidth and module for strength requirements only. Take service 	factor = 1.25.

(OR)

19.	Design the face width for a steel helical gear pair based on transverse module using following 	data:

Power transmitted = 30 kW, speed of pinion = 1500 rpm.

Pressure angle in diametral plane = 20full depth involute.

Helix angle = 30, velocity ratio = 4 : 1,

Number of teeth on pinion = 24.

Static stress for the gear pair material = 50.7 M.Pa.

B.H.N for gear material = 350.

20.	A pair of straight tooth bevel gears at acute angles are used to transmit  5 kW at 900 rpm of the 	pinion. The shaft angle is 60 and the speed reduction is 3 : 1. The teeth are 20 full depth form 	and the number of teeth on pinion is 20. The pinion is to be made of steel whose allowable 	static stress is 120 MPa. The gear is to be made of cast iron whose allowable static stress is         	80 MPa. The load is steady and the service is continuous. Determine the face width of the gears 	from the stand point of strength only.

(OR)

21.	The following data refer to a worm and worm gear drive:

Centre distance = 200 mm, pitch diameter of worm = 80 mm. No. of starts =3, Axial module = 8. 	Transmission Ratio = 20. Bending strength of phosphor bronze (worm gear) = 55 MPa. Tooth 	form = 20 full depth involute. Determine:

a.	Number of teeth on the worm gear.

b.	Lead angle

c.	Face width of the worm gear to transmit 15 kW of power at 1750 rpm of the worm, based on 		beam strength of the gear.

22.	An automotive gear box gives three forward speeds and reverses with a top gear of unity and 	bottom and reverse gear ratio of approximately 3.3 :1. The centre distance between the shafts is 	to be 0.11 m approximately. Gear teeth of module 3.25 are to be employed. Obtain the required 	gear ratios and the respective no. of teeth on them.

(OR)

23.	A driving shaft which is supporting a three stepped, pulley, rotates at a Constant speed of

160 	rpm.  It is connected by a cross belt to a parallel shaft, which is at a distance of 0.72m from 	it. 	The follower shaft which is supporting another similar stepped pulley has to run at 60, 80 	and 	100 rpm. The smallest pulley on the driving shaft is of 80mm diameter. Determine the 	diameters of the remaining two stepped pulleys.

24.	Present step – by – step general procedure to design a ‘circular arc cam’ with ‘Flat faced 	follower’ for any one position of the follower in contact with the cam.

(OR)

25.	A 2 –start power screw with ISO metric trapezoidal threads, is used to raise a load of 300 kN. 	The nominal diameter is 100mm and the pitch is 12mm. The co-efficient of friction at screw 	thread is	0.15. Neglecting collar friction, calculate:

a.	Torque required to raise the load.

b.	Torque required to lower the load and	c.	Efficiency of the screw.