Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MASS TRANSFER OPERATIONS							Time : 3 hours

Subject Code:	BT236				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the unit of Molecular diffusivity and Eddy diffusivity.

2.	What type of diffusion is taking place in distillation?

3.	Define Reflux ratio.

4.	What is an azeotrope?

5.	What is HETP?

6.	At minimum Ls/Gs the number of theoretical plates is equal to -----------.

7.	Hyper sorption refers to ------------- process.

8.	Rate of adsorption increases as the -------------.

9.	Coffee is prepared from coffee seeds by leaching with --------------.

10.	In LLE, if selectivity (β) is unity then no separation takes place (True/False).

PART – B  (5 x 3 = 15 MARKS)

11.	State and explain Fick’s law of diffusion.

12.	Explain Flash Distillation.

13.	What are the different types of packing? Explain briefly?

14.	What are the essential characteristics of a good adsorbent?

15.	Explain the factors to be considered in the choice of solvent for an extraction system.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive an expression for diffusion of gas A through non diffusing gas B.

(OR)

17.	Methane diffuses at a steady state through a tube containing Helium at point 1, the pressure is 15.55 KPa. At point 2, which is 30 mm apart, the partial pressure of Methane is10 kPa. The total pressure is 101.32 KPa and the temperature is 293K. At this pressure and temperature the diffusivity is 6.75*10-5 m2/s. calculate

a.	The flux of methane at steady state for equimolal counter diffusion.

b.	The partial pressure at 2 cm from point 1.

18.	Derive the Rayleigh’s equation.

(OR)

19.	With a neat flow chart explain extractive distillation

20.	Explain the concept of NTU and HTU.

(OR)

[P.T.O]

21.	A soluble gas is absorbed in water using a packed tower. The equilibrium relationship may be taken as Ye=0.06Xe where Ye and Xe are ratio’s of moles of inert component. The terminal conditions are

|  | TOP | BOTTOM |
| --- | --- | --- |
| X | 0 | 0.08 |
| Y | 0.001 | 0.009 |

If the individual heights of transfer units based on liquid and gas phases respectively are   Hx=0.24 m and Hy=0.36 m, what is the height of the packed section?

22.	With a suitable diagram explain fixed bed adsorber.

(OR)

23.	Explain the principles and equilibria of ion exchange operation.

24.	Explain Type I extraction system with suitable example.

(OR)

25.	Illustrate the working principles of Bollman extractor with a suitable diagram.