Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MICROPROCESSORS										Time : 3 hours

Subject Code:	CA242				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How many pins does the 8085 Microprocessor IC have?

2.	What is microprocessor?

3.	What is the word length of 8085 microprocessor?

4.	Explain the function of IO/M in 8085.

5.	Define mnemonics.

6.	What is T-state

7.	What are software interrupts?

8.	What is polling?

9.	What is baud rate?

10.	What is USART?

PART – B  (5 x 3 = 15 MARKS)

11.	What is Tri-state logic? Why it is needed in microprocessor system?

12.	What are the operations performed by ALU of 8085?

13.	List the various machine cycles of 8085.

14.	What are the advantages of subroutine?

15.	What are the operating modes of port –A of 8255?

PART – C  (5 x 15 = 75 MARKS)

16.	Compare micro, mini and large computers

(OR)

17.	a.	List the advantages and disadvantages of microprocessor based system 					(7)

b.	Explain about assembly language					   								(8)

18.	Give a detailed view of the 8085 architecture and mention the functions of various blocks.

(OR)

19.	Explain the basics of memory interfacing and address decoding

20.	Draw the flowchart to find the maximum data in a data set. The size of the data set is 0A16 and is stored in memory location 200016. The first data is contained in memory location 200116. Assume that the numbers in the data set are all 8-bit unsigned binary numbers. Convert the flowchart to an ALP.

(OR)

21.	Draw and explain the timing diagram of memory read cycle

22.	Describe different types of interrupts available in 8085

(OR)

23.	Explain the control word format of 8253 counter and its different modes of operations

24.	a.	What is a DMA? List its merits and demerits compared to other data transfer schemes.		(5)

b.	With neat diagram explain the DMA transfer scheme.								     (15)

(OR)

25.	Draw and explain the internal block diagram for 8257.