Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FLUID MECHANICS AND MACHINERIES					Time : 3 hours

Subject Code:	CE212			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by the term ‘fluid kinematics’?

2.	What is the unit of Dynamic Viscosity’?

3.	What are the two methods of describing fluid motion?

4.	Write down the Cauchy – Riemann equation.

5.	What are the practical applications of Bernoulli’s Equation?

6.	How will you determine loss of head due to obstruction in pipe?

7.	What is ‘NPSH’ related to centrifugal pump.

8.	On what principle momentum equation is derived.

9.	How will you determine the number of buckets for a pelton wheel turbine?

10.	What is the range of specific speed for propeller and Kaplan turbine?

PART – B  (5 x 3 = 15 MARKS)

11.	Derive an expression for capillarity.

12.	How will you distinguish stream line, streak line and path line?

13.	What do you understand by the term ‘water hammer?

14.	What is an air-vessel and what is it’s purpose?

15.	What is a surge tank? Where is it used?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A cylindrical shaft of 90mm diameter rotates about a vertical axis inside a fixed cylindrical

tube of length 50cm and 95mm internal diameter. If the space between the tube and the shaft is filled by a lubricant of dynamic viscosity 0.2 N – s/m2, determine the power required to overcome viscous resistance, when the shaft is rotated at a speed of 240 rpm.		(12)

b.	If the surface tension at the air-soap interface is 0.088 N/m, calculate the internal pressure in

a soap bubble of 30mm diameter.												(3)

(OR)

17.	a.	Derive an expression for rate of increase of pressure in a vertically downward direction in a

static fluid.																	(8)

b.	A circular plate of diameter 75cm is immersed in a liquid of relative density 0.80 with its

plane making an angle of 30 with the horizontal. The centre of the plate is at a depth of

1.5 m below the free surface. Calculate the total force on one side of the plate and the location of the centre of pressure.												(7)

18.	A cylindrical vessel closed at the top and bottom is 0.24m in diameter, 1.44m. long and contains

water upto a height of 0.96m.

- Find the height of the paraboloid formed, if it is rotated at 480 r.p.m about its vertical Axis.

- Find the speed of rotation of the vessel, when axial depth of water is zero.

(OR)

[P.T.O]

19.	The velocity vector in an incompressible flow is given by

V = (6xt + yz2) i + (3t + xy2) j + (xy – 2xyz – 6tz) k

a.	Verify whether the continuity equation is satisfied.

b.	Determine the acceleration vector at point A (1, 1, 1 ) at t = 1.0.

20.	a.	Derive an expression for the determination of actual discharge through a venturimeter. (8)

b.	Find the discharge through a trapezoidal notch which is 1.2m wide at the top and 0.5m at the

bottom and is 0.4m in height. The head of water on the notch is 0.3m. Assume cd for rectangular portion =0.62, while for triangular portion = 0.60.

(OR)

21.	A pipeline carrying water has a diameter of 0.5m and is 2km long. To increase the delivery another pipeline of the same diameter is introduced parallel to the first pipe in the second of its length. Find the increase in discharge if the total head loss in both the cases is 15m. Assume         f = 0.02 for all the pipes.

22.	A centrifugal pump impeller has an outer diameter of 30cm and an inner diameter of 15cm. The pump runs at 1200 rpm. The impeller vanes are set at a blade angle of 30 at the outlet. If the velocity of flow is constant at 2.0 m/s, calculate

a.	the velocity and direction of water at outlet.

b.	the head developed by assuming a manometric efficiency of 0.85 and

c.	the blade angle at the inlet.

(OR)

23.	A single acting reciprocating pump has the following data.

Cylinder diameter = 10 cm		:	stroke = 25 cm

Static suction head = 4.0m		:	Dia. of suction pipe = 5.0 cm

Suction pipe length = 6m		:	Crank speed = 30 rpm

Delivery pipe diameter = 5 cm	:	Delivery pipe length = 25m

Static delivery head = 16.0m

Estimate the pressure head on the piston at the

- beginning of suction and delivery stroke.

- Middle of suction and delivery stroke.

- End of suction and delivery stroke.

Assume atmospheric pressure = 10.0 m of water (abs) and friction factor f = 0.02.

24.	A pelton wheel 2.5m diameter operates under the following conditions.

Net available head 	=	300m

Speed				=	300 rpm

Coefficient of velocity of jet	=	0.98

Blade friction coefficient	=	0.95

Blade angle				=	165

Diameter of the jet		=	20 cm

Mechanical efficiency		=	0.95

Determine (a) 	The power developed			    (b)	Hydraulic efficiency

(c)	Specific speed

(OR)

25.	A Kaplan turbine develops 15000 kW power at a head of 30m. The diameter of the boss is 0.35 times the diameter of the runner. Assuming a speed ratio of 2.0, a flow ratio of 0.65 and an over all efficiency of 90% calculate.

a.	diameter of the runner				b.	rotational speed		c.	specific speed