Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	BASIC STRUCTURAL DESIGN								Time : 3 hours

Subject Code:	CE235			       						               	 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is working stress method?

2.	Define thermal load

3.	What is non load bearing wall?

4.	What is solid wall?

5.	What is bed block?

6.	Define middle third rule

7.	What is permissible stress?

8.	Define flitched beam

9.	Differentiate butt and fillet weld

10.	Write the permissible stress of a welded joint subjected to shear and torsion.

PART – B  (5 x 3 = 15 MARKS)

11.	What are strength, stability and stiffness?

12.	Explain the design procedure of axially loaded wall

13.	Explain the concept of the design or reinforced brick lintels

14.	Differentiate solid and built up column

15.	Explain splicing joint.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the choice of different structural material for different applications.

(OR)

17.	Explain the concept of calculation of wind load and seismic load for structure.

18.	A masonry column is subjected to an axial load of 400kN with an axial eccentricity of 50 mm. The column in hinged at both ends. The height of the column is 5m. Design the column for square and rectangular cross sections.

(OR)

19.	A masonry wall is subjected to an axial load of 100kN and bending moment of 20kNm. The height of the wall is 4m. Design the wall.

20.	Explain the concept of the design of spread footing for walls and columns.

(OR)

21.	Design an axially loaded reinforced masonry column for a load of 700kN.The height of the column 6m. The column is fixed at both ends.

22.	Design a simply supported timber beam of span 6m subjected to a load of 20 kN/m (Excluding self weight) throughout the beam. Apply necessary checks.

(OR)

[P.T.O]

23.	A timber column is subjected to an axial load 2400 kN. The height of the column is 3m. The column is hinged at both ends. Design it as a built up column. Apply usual checks.

24.	A welded crane bracket supports a load of 150kN at a distance of 120 mm from the edge of a column ISWB 300. Using 15 mm thick plates, design the eccentrically loaded bracket connection.

(OR)

25.	Design a bolted connection to support the beam ISMB 300 which transmits a load of 200 kN to the flange of the column ISWB 400.