Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF DEFORMABLE BODEIS - I				Time : 3 hours

Subject Code:	CE240		       						               		 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Hooke’s law?

2.	Relation between Elastic modulus (E) and Bulk modulus (K) is ____________.

3.	If ex and ey be the strains in mutually perpendicular direction then the strain in any direction is _____________.

4.	The thin cylinders are frequently required to operate under pressure upto _____________MN/m2

5.	Draw Shear force and Bending Moment diagram for the cantilever beam with point load (W) at the free end, the length of the beam is ‘l’.

6.	Define point of contraflexure.

7.	Write the bending equation.

8.	What is the ratio of maximum shear stress to the average shear stress in the case of solid circular 	section?

9.	What is the polar moment of Inertia for a hollow circular shaft?

10.	Closed coil spring subjected to axial load, what is the stiffness of the spring?

PART – B  (5 x 3 = 15 MARKS)

11.	Find the modulus of rigidity of the material, the young’s modulus and poisson’s ratio is                  1.25 X 105 N/mm2 and 0.34.

12.	Define principle stresses and principle plane.

13.	Explain different types of supports.

14.	Derive the expression for the section modulus and moment of resistance for the hollow circular section.

15.	A closely coiled helical spring is to carry a load of 500 N. Its mean coil diameter is to be 10 times that of the wire diameter. Calculate these diameters if the maximum shear stress in the material of the spring is to be 80 MN/m2

PART – C  (5 x 15 = 75 MARKS)

16.	A steel rod of 20 mm diameter passes centrally through a copper tube of 50 mm external diameter and 40 mm internal diameter. The tube is closed at each end by rigid plates of negligible thickness. The nuts are tightened lightly home at the projecting parts of the rod. If the temperature of the assembly is raised by 50° C. Find the stresses in the rod and the tube. Assume modulus of elasticity for steel and copper as 200 k N/mm2and 100 k N/mm2respectively. The coefficient of extension for steel is 12 X 10-6° C and for copper, it is 18X 10-6° C

(OR)

17.	Calculate the modulus of elasticity and Poisson’s ratio of a metallic bar of length 30 cm, breadth 4 cm and depth 4 cm when the bar is subjected to an axial compressive load of 400 kN. The decrease in length is given as 0.075 cm and increase in breadth is 0.003 cm.

18.	A cylindrical thin drum 800 mm in diameter and 4 m long is made of 10 mm thick plates. If the drum is subjected to an internal pressure of 2.5 Mpa. Determine its change in diameter and length. Take E as 200 GPa and Poisson’s ratio as 0.25.

(OR)

19.	At a point in a bracket the stresses on two mutually perpendicular planes are 400 M N/m2tensile and 300 M N/m2 tensile. The shear stress across these planes is 200 M N/m2. Determine the magnitude and direction of principal stresses and maximum shear stress.

20.	Calculate the reactions, Shear force and Bending Moments on the beam shown in Fig. Draw the shear force and bending moment diagram.

20 KN.m

20 KN

30 KNm

A

B

C

D

E

1.5 m

0.5 m

1 m

1 m

(OR)

21.	Draw Shear force and bending moment diagrams for the following beam and show the points of maximum bending moment. Also find the points of Contra flexure.

2 KN

3 KN

6 KN

3 KN/m

A

E

B

C

D

2.4 m

1.5 m

1.5 m

1.5 m

22.	An I section beam 290 X 200 mm has a web thickness of 12 mm and a flange thickness of 20 mm. It carries a shearing force of 80 kN at a section. Find the maximum shearing stress developed and sketch the shear stress distribution diagram.

(OR)

23.	a.	A cantilever beam of cross section 100 mm wide and 200 mm deep carries a point load of    	10 kN at the free end of the beam. The bending stress is not to exceed 50  N/mm2. Find the 	span of the cantilever beam.												            (10)

b.	Write the assumptions made in theory of simple bending								(5)

24.	A stepped shaft is subjected to a torque as shown in fig. The maximum shear stress in the shaft is limited to 80 Mpa. What is the total angle of twist at the free end. Take C = 84 GPa.

2 T

P

150 mm

100 mm

R

Q

T

1000 mm

1500 mm

(OR)

25.	A close coiled helical spring is to have a stiffness of 2 N/mm of compression under a load of    80 N and a maximum shearing stress of 140 N/mm2 . The solid length of the spring is to be 50mm. Find the diameter of the wire, the mean diameter of the coils and the number of coils required. C = 42 X 103 N/mm2