Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF DEFORMABLE BODIES - II				Time : 3 hours

Subject Code:	CE241			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of Conjugate beam method?

2.	The deflection by moment area method is given by __________.

3.	Define a column.

4.	Define crippling load.

5.	Write the formula for finding the radial pressure in a thick cylinder having a radius “x”.

6.	What is the assumption made in determining the distribution of stresses in the cylinder?

7.	Define shear centre.

8.	What is centroid?

9.	Write any one assumption made in the theory of simple bending.

10.	Maximum principal strain theory also known as ___________

PART – B  (5 x 3 = 15 MARKS)

11.	A beam of uniform rectangular section 200mm wide and 300mm deep is simply supported at its

ends. It carries a uniformly distributed load of 9kN/m run over the entire span of 5m.If the value of E for the beam material is 1x104N/mm2.Find the slope at the supports and maximum deflection.

12.	A solid round bar 3m long and 5cm in diameter is used as a strut with both ends hinged. Determine the crippling load. Take E=2x105N/mm2.

13.	Determine the maximum and minimum hoop stress across the section of a pipe of 400mm internal diameter and 100mm thick when the pipe contains a fluid at a pressure of 8N/mm2.

14.	What are the assumptions made in Winkler bach theory?

15.	A rectangular column of width 200mm and of thickness 150mm carries a point load of 240kN at an eccentricity of 10mm.Determine the maximum and minimum stresses on the section.

PART – C  (5 x 15 = 75 MARKS)

16.	A cantilever of length 3m carries a uniformly distributed load of 80kN/m over the entire length. If E=2x108kN/m2 and I=108mm4 find the slope and deflection at the free end using conjugate beam method.

(OR)

17.	Find the slope and deflection of a simply supported beam carrying a point load ‘W’at the centre by moment area method. The length of the beam is ‘l’.

18.	Determine Euler crippling load for an I section joist 40cmx20cmx1cm and 5m long which is used as a strut with both ends fixed. Take young’s modulus for the joist as 2.1x105N/mm2.

(OR)

[P.T.O]

19.	Determine the maximum stress induced in a horizontal strut of length 2.5m and of rectangular cross section40mm wide and 80mm deep when it carries an axial thrust of 100kN and a vertical load of 6kN/m length. The strut is having pin joints at its ends. Take E=208GN/m2.

20.	A thick spherical shell of 200mm internal diameter is subjected to a internal fluid pressure of 7N/mm2.If the permissible tensile stresses in the shell material is 8N/mm2, find the thickness of the shell.

(OR)

20.	The principal stresses at appoint in an elastic material are 200N/mm2(tensile), 100N/mm2 (tensile)and 50N/mm2(compressive).If the stress at the elastic limit in simple tension is 200N/mm2,determine whether the failure of the material will occur according to maximum principal strain theory. Take poisson’s ratio 0.3.

22.	A ring is made of stock with a circular cross section 8cm in diameter. The inside diameter of the ring is 10cm. the load is 20kN.Calculate the stresses at A and B.

(OR)

23.	Locate the shear centre of the cross section shown in figure.

2cm

1cm

15cm

4cm

3cm

24.	A rectangular beam 6cmx4cm is 2m long and is simply supported at the ends. It carries a load 1kN at mid span. Determine the maximum bending stress induced in the beam.

(OR)

25.	A rolled steel joist 450mmx200mm is freely supported on a span of 4.5m. The flanges are strengthened by two 300mmx20mm plates one riveted to each flange. The second moment of area of the joist is 35,060cm4. Calculate the greatest central load that will carry if the stress in the plates is not to exceed 120MPa.