Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ENGINEERING MECHANICS								Time : 3 hours

Subject Code:	CE242			       						                       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Two forces 3N and 1N act on a particle. The Resultant when they act right angle is --------; resultant when they act at 60° is ---------.

2.	Define Resultant force.

3.	The resultant of a coplanar concurrent force system will have system will have the same moment as the moment of each force and is known as ---------------- theorem.

4.	Frictional force is -----------------of area of contact.

5.	Define Centroid.

6.	State parallel axis theorem.

7.	A car travels between two stations which are 15 km apart in 20 minutes. If the car accelerates for a part of journey uniformly followed by uniform retardation, the maximum speed attained by the car during the journey is --------------------------.

8.	What do you mean by projectile?

9.	When a body slides down an inclined surface, the acceleration of the body is ----------------.

10.	Mechanical efficiency (η) is -------------------------

PART – B  (5 x 3 = 15 MARKS)

11.	State parallelogram law of forces.

12.	Find the ratio of tension in tight and slack sides of belt with the contact angle of 160°.Assume     μ = 0.18.

13.	Distinguish between centroid and centre of gravity.

14.	A stone is dropped into a well and it the splash is heard 2.50 seconds later, determine the depth of water surface assuming that the velocity of sound is 330 m/s.

15.	Define linear momentum and angular momentum.

PART – C  (5 x 15 = 75 MARKS)

16.	Three smooth pipes each weighing 20KN and of diameter 60 cm are to be placed in a rectangular channel with horizontal base as shown in Fig 1. Calculate the reactions at the points of contact between the pipes and between the channel and the pipes. Take width of channel as 160 cm.

[P.T.O]

Fig 1

(OR)

17.	The tension in the supporting cable AB is 10 KN (Fig 2). Write the force which the cable exerts on the beam BC as a vector T. Determine the angle of T form with the positive x, y and z axes.

Fig 2

18.	Determine the support reactions of the beam shown in Fig 3.

Fig 3

(OR)

19.	Two blocks of mass 20 kg and 40 kg are connected by a rope passing over a friction less pulley as shown in Fig 4. Assuming coefficient of friction as 0.3 for all contact surfaces find the tension in the string, acceleration of the system. Also compute the velocity of the system after 4 seconds starting from rest.

Fig 4

20.	Find the moment of inertia of an un symmetrical I section shown in Fig 5. about its centroidal axes.

Fig 5

(OR)

21.	Determine the mass moment of inertia of the composite body about Z- axis shown in Fig 6.The mass density of the cylinder is 6000 kg/m3 and the rectangular prism is 7000 kg/m3.

Fig 6

22.	A train is traveling from A to D along the track shown in Fig 7. Its initial velocity at A is zero. The train takes 5 min to cover the distance AB, 2250 m length and 2.5 minutes to cover, the distance BC, 3000 m in length, on reaching the station C, the brakes are applied and the train stops 2250 m beyond, at D (i) Find the retardation on CD, (ii) the time it takes the train to get from A to D, and (iii) its average speed for the whole distance.

Fig 7

(OR)

[P.T.O]

23.	An airplane is flying on a straight level course at 200 km/hr at a height of 1000 metre above the ground. An anticraft gun located on the fires a shell with an initial velocity of 300 m/s, at the instant when the plane is vertically above it. At what inclination, to the horizontal, should the gun be fired to hit the plane? What time after firing, the gun shell will hit the plane? What will then be the horizontal distance of the plane from the gun?

24.	Two blocks A and B of 70 weight 100 N and 200 N respectively are initially at rest on a 30° inclined plane as shown in Fig 8. The distance between the blocks is 6 m. The co efficient of friction between the block A and the plane is 0.25 and that between the block B and the plane is 0.15. If they released at the same time, in what time the upper block (B) reaches the lower Block (A).

Fig 8

(OR)

25.	A spring is used to stop a 70 kg package which is moving down a 30° inclined as shown in Fig 9. The spring constant is 200 N/m. The package is released from rest when it is 2m from the spring. The package strikes the spring which is compressed to 200 mm, before the package is brought to rest. Determine the coefficient of kinetic friction between the package and the plane

Fig 9