Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	SURVEYING – I											Time : 3 hours

Subject Code:	CE243				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The Leader and the Follower in the chain surveying the two are together called the _______.

2.					 This is for __________

3.	It is a Line means the angle between a certain chosen Fixed distance and the line. What is it?

4.	_______ is a line is the projection of the length of the line on the meridian.

5.	_______ of a point is the elevation of the point above the adopted datum.

6.	This is the first staff reading taken after setting up the instrument at a station. What is it?

7.	A circular plat is provide beneath the vernier plate is called the ________

8.	In the modern instrument horizontal hairs are provided equidistant from the main horizontal hair.

These additional hairs are called stadia hairs. What is the additional hair are needed?

9.	This is the actual length of the curve touching the end straights, between the point of curve and the point of tangency. What is it?

10.	Mid – ordinate is also called the versed sine of the curve in _______

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between whole circle bearing and Reduced bearing.

12.	Name the various accessories in plane table surveying.

13.	The areas enclosed by contour lines for a proposed dam site are given below.

| Contour level (m) | Area enclosed by the contour ( hectares) |
| --- | --- |
| 520 | 18 |
| 525 | 96 |
| 530 | 320 |
| 535 | 680 |
| 540 | 895 |

Find the volume  between the given extreme contours by prismoidal formula.

14.	The diaphragm of a tachometer has three cross hairs spaced at 1.2mm. The Focal length of the object glass is 240mm. The distance between the object glass and the trunnion axis is 115mm. determine the constants of the tachometer.

15.	What is the relation between the degree of a curve and its radius?

PART – C  (5 x 15 = 75 MARKS)

16.	A 20 m tape standardized at 280.5c was used to measured a survey line by stretching it on the sloping ground. The measured distance was 602.5m at a field temperature of 180C and the following slopes were observed. 30 12 for 100m, 20 48 for 125m, 2000 for 100m 50 2 for 150m 40 8for 75m, 60 10 for 52.5m. Find the true horizontal distance. Take α=12×10−6/0c.

(OR)

17.	Describe the various types of obstacles in chaining.

[P.T.O]

18.	Describe the sources of error in compass surveying.

(OR)

19.	State the two point problem. Explain how the table is oriented at a station by two point problem.

20.	The distance between two stations A and B is 50m. A dumpy level was placed at C on an extension on AB such that Ac = 75m. The following observations were made. Staff on stations A ( RL.22 . 850m.) = 1.255m, staff on station B(RL.23.700m) = 0.480 m.

a.	Is the line of collimation tilted upwards/downwards and by how much?

b.	Find the correct staff readings that should be obtained on A and B.

(OR)

21.	An embankment for a road is to be having a formation width of 10m and side slopes of 1+0.1 and an average height of 18m. The embankment has to be provided at a gradient of 1 in 50 from contour 260 m to contour 610m. The transverse slope of the ground is 12 to 1 at right angles to center line. Find the length, volume of the embankment of the road and cost of Earth work @ Rs.200/ 1000 cum.

22.	Explain in briefly the permanent adjustment of a Transit theodolite.

(OR)

23.	Discuss the common Errors theodolite measurements.

24.	Two straights intersect at a deflection angle of 800 and are connected by a circular curve of radius 10 chains. Find

a. Length of each end tangent		b. Length of the curve		c. Length of the long chord

d. Apex distance		e. Mid ordinate of the curve	f. Degree of the curve.

(OR)

25.	How is vertical curve determined in highways by sight distance method? Explain.