Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	SURVEYING - II									               Time : 3 hours

Subject Code: 	CE244			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the values of multiplying constant and additive constant used in Stadia Tacheometry.

2.	In what way the use of anallactic lens in tacheometry, significant?

3.	What is meant by satellite station?

4.	What is meant by pull for normal tension?

5.	Distinguish between true error and residual error.

6.	What is meant by systematic error.

7.	What is meant by equatorial radius?

8.	What is meant by Nautical mile?

9.	What is meant by tiet?

10.	What is meant by hydrographic surveying?

PART – B  (5 x 3 = 15 MARKS)

11.	The focal length of the lens used in tacheometric survey is 0.3m and the multiplying constant is 100. Find the distance between the stadia hairs.

12.	Given  = 41 15 20  weight 3,    = 28 20 40 weight 1 find the weight of  + .

13.	What is reconnaissance? Describe the process of reconnaissance for triangulation.

14.	What is a spherical triangle? Discuss its properties.

15.	What are the methods of measuring co-ordinates of photo images?

PART – C  (5 x 15 = 75 MARKS)

16.	Deduce expressions for horizontal distance and elevation for all the cases relating to tangential tacheometry.

(OR)

17.	A tacheometer fitted with an anallactic lens was used to observe the following:

From		To		Bearing		Vertical angle		  Hair  readings

C			A		    40			+12			1.255, 2.090, 2.925

B		    310			+10			1.030, 2.505, 3.980

The value of constant was 100 and the staff was held vertically. Determine the length, bearing and gradient of AB.

18.	The altitude of two proposed stations A and B 130 km apart are respectively 220m and 1160m. The altitude of the two points C and D on a profile between them are respectively 308m and 623m. The distance AC = 50 km and AD = 90 km. Determine whether A and B are intervisible and if necessary find the minimum height of a scaffolding at B assuming A as the ground station such that the new line of sight clears the obstruction by at least 2m.

(OR)

[P.T.O]

19.	P is the main station. S its satellite station. A and B are two known triangulation stations. How would you find the true angle APB from measurements at S, the satellite station?

20.	Find the most probable values of the angles A, B, C and D from the following observations closing the horizon at a station.

A = 122 05 58.6		weight 2

B =   86 45 16.4		weight 1

C =   72 50 31.2		weight 3

D =  78 18 16.6		weight1

(OR)

21.	Adjust the following observations using the method of least square

A 		= 25 17 10.2			weight 1

B 		= 28 22 16.4			weight 2

C 		= 32 40 28.5			weight 3

A + B		= 53 39 23.1			weight 2

A + B + C	= 86 19 57.8			weight 1

22.	What are the methods of determining azimuth? Discuss any one of them.

(OR)

23.	Prove that the latitude of a place is equal to the altitude of the pole.

24.	Discuss method of determining ground co-ordinates from the photo coordinates.

(OR)

25.	Discuss the following:

a.	Mirror stereoscope		b.	Lens stereoscope