Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF FLUIDS									Time : 3 hours

Subject Code:	CE245			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Specific gravity is also called relative density. (True or False)

2.	When the fluid is at rest, the shear stress is------------------------

3.	For incompressible flow, density is equal to -----------------------

4.	If the Reynolds number is more than 4000, it is called turbulent flow. (True or False)

5.	Venturimetre is a device used for measuring the ---------------------- of a fluid flowing through a pipe.

6.	The sheet of water flowing through a notch (or) over weir is called crest.(True or False)

7.	A long bent pipe which is used to transfer liquid from a higher elevation to a lower level is called ---------------------.

8.	The line which gives the sum of pressure head and datum head of a flowing fluid in a pipe with respect to some reference line is called -----------------

9.	Dimensional analysis is performed by two methods namely ---------and-----------

10.	Kinematics similarity means the similarity of forces between the model and prototype (True or False)

PART – B  (5 x 3 = 15 MARKS)

11.	State the Newton’s law of viscosity and classify Newtonian and Non- Newtonian fluids.

12.	Difference between forced vortex and free vortex flow.

13.	The head of water over the centre of an orifice of diameter 20 mm is 1m. The actual discharge through the orifice is 0.85 liter/sec. Find the co- efficient of discharge.

14.	List out the cases of minor loss of energy in flow through pipes.

15.	How are the repeating variables selected for dimensional analysis?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define the term “buoyancy” and “centre of buoyancy”.                      				(3 )

b.   Determine the total pressure and centre of pressure on an isosceles triangular plate of base

4m and altitude 4m, when it is immersed vertically in oil of specific gravity 0.90. The base of the plate coincides with the free surface of oil.									(12)

(OR)

17.	A 15 cm diameter vertical cylinder rotates concentrically inside another cylinder of diameter 15.1 cm. Both cylinders are 25 cm high. The space between the cylinders is filled with a liquid whose viscosity is unknown. If a torque of 12 N-m is required to rotate the inner cylinder at 100 rpm, determine the viscosity of the fluid.

18.	a.	The following cases represent the two velocity components; determine the third component

of velocity such that they satisfy the continuity equation.

(1) u = x2+y2+z2  ; v =  xy2- yz2 + xy

(2) v = 2y2,  w = 2xyz                                                                      				 (10)

[P.T.O]

b. 	What are the forces are present in the fluid flow?                              				   (5)

(OR)

19.	A pipe of diameter 400mm carries water at a velocity of 25m/s. The pressures at the points A and B are given as 29.43 N/cm2 and 22.563 N/ cm2 respectively while the datum head at A and B are 28m and 30m . Find the loss of head between A and B.

20. 	Derive an expression for rate of flow through venturimeter.

(OR)

21.	a.	Draw a sketch of Reynolds experiment apparatus and mention its parts.       			 (5 )

b.	Water is flowing through a rough pipe of diameter 600 mm at the rate of 600 lit/sec. The

wall roughness is 3mm. Find the head loss due to friction, if the length of pipe is 1000m.

(10)

22.	The rate of flow of water through a horizontal pipe is 0.25 m3/s. The diameter of the pipe which  is 200mm is suddenly enlarged to 400mm. The pressure intensity in the smaller pipe is         11.772 N/cm2. Determine loss of head due to sudden enlargement, pressure intensity in the larger pipe and the power loss due to enlargement.

(OR)

23.	a.	Prove the condition for maximum transmission of power is H=3 hf.                       	 (9 )

Prove that the maximum efficiency of transmission of power is 66.70%.                	  (6)

24. 	Determine the dimensions of the quantities for angular velocity, angular acceleration, discharge,

kinematic viscosity, force and specific weight.

(OR)

25.	a.	 State Buckingham’s π- theorem.                                                     					  (3)

b.	 Write short notes on dimensionless numbers.                                 					 (3 )

c.    In a 1 in 20 model of stilling basin, the height of the hydraulic jump in the model is

observed to be 0.20m. What is the height of the hydraulic jump in the prototype? If the energy dissipated in the model is 0.10 kW, what is the corresponding value in prototype? (9)