Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	APPLIED HYDRAULICS AND FLUID MACHINES	               Time : 3 hours

Subject Code: 	CE246			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What are the conditions for a trapezoidal channel section to be most economical?

2. 	Froude number is ____________ for a subcritical flow.

3. 	Boundary layer theory was given by _____________.

4.	What is meant by total drag force on a object?

5.	What do you mean by efficiency of a jet?

6.	Write the expression for force exerted by a jet on the centre of moving curved vane.

7. 	What do you mean by governing of turbines.

8. 	Differentiate Reaction and Impulse turbines.

9. 	What do you mean by ideal indicator diagram?

10.	Give the classification of pumps with examples.

PART – B  (5 x 3 = 15 MARKS)

11.	Define minimum specific energy and state the formula to calculate it.

12.	What are the methods available to prevent boundary layer separation?

13. 	Write the equations to calculate horizontal and vertical components of force in a bend.

14. 	What is meant by draft tube? State its uses.

15. 	Define Thoma’s Cavitation factor.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	A trapezoidal channel has side slopes of 3 horizontal to 4 vertical and slope of its bed is 1 in

2000. Determine the optimum dimensions of the channel, if it is to carry water at 0.5m3/s. Take Chezy’s constant as 80.													(12)

b. 	Explain Backwater and Drawdown curves.										(3)

(OR)

17.	a. 	Derive the relationship between the sequent depth and initial depth in a hydraulic jump. (3)

b.	A sluice gate discharges water into a horizontal rectangular channel with a velocity of 10m/s

and depth of flow of 1m. Determine the depth of flow after the jump and consequent loss in total head.																	(12)

18.	a.	Find the ratio of displacement thickness to boundary layer thickness and momentum

thickness to boundary layer thickness for the velocity distribution in the boundary layer given by 																	(12)

u/U = 3/2( y/δ) – ½( y/δ)2

Where, u = velocity in boundary layer at a distance y, U = Free stream velocity and               δ = boundary layer thickness

b. 	Define Laminar Sub-layer.														(3)

(OR)

[P.T.O]

19.	a. 	A thin plate is moving instill atmospheric air at a velocity of 4 m/s. The length of the plate is

0.5m and width 0.4m. Calculate the (i) thickness of the boundary layer at the end of the plate (ii) drag force on one side of the plate. Take density of air as 1.25 kg/m3 and kinematic viscosity 0.15 stokes.															(7)

b. 	A body of 1.5m length with projected area 1.2 m2 normal to the flow direction is moving

through a water. Find the drag force on the body assuming CD = 0.5, μ = 0.01 poise. Take Reynolds Number = 80x105.													(8)

20.	a. 	State the condition for attaining maximum efficiency for a jet falling on a series of vane and

prove it.																		(10)

b. 	A jet of water coming out from a nozzle of 25 mm with a velocity of 25 m/s strikes a flat

plate which is moving with a velocity of 5m/s in the direction of jet. Determine the thrust when the plate (a) is vertical (b) inclined at 60o to the jet.							(5)

(OR)

21.	a. 	Derive Impulse Momentum equation and state its applications.						(3)

b. 	250 litres/s of water is flowing in a pipe having a diameter of 300 mm. If the pipe is bent by

135o(that is change from initial to final direction is 135o), find the magnitude and direction of the resultant force on the bend. The pressure of water flowing is 39.24 N/cm2. 	(12)

22.	a. 	A pelton wheel has a mean bucket speed of 10m/s with a jet of water flowing at the rate of

700 l/s under a head of 30m. The buckets deflected the jet through an angle of 160o. Calculate the power given by water to the runner and hydraulic efficiency of the turbine. Assume Cv = 0.98.															(12)

b. 	Explain the principle of Jet Pump with neat diagram.								(3)

(OR)

23.	a.	The inner and outer diameter of an inward flow reaction turbine are 100cm and 80cm

respectively and its breadth at inlet is 20cm. 10% of the flow area is blocked by the blade thickness. The flow velocity at the inlet is 2.5m/s and runner speed is 200rpm. The guide vanes at the inlet make an angle of 12o to the wheel tangent.						(10)

b.	Explain the performance curves of turbine.										(5)

24.	a.	What is an Airvessel and what are its functions?									(3)

b.	A single acting reciprocating pump has a piston diameter of 12cm and a stroke of 24cm is delivering water to the tank which is 10m above the centre of the pump. The pump is located 5m above the level of the sump. The diameter and length of the suction pipe are 5 cm and 5m respectively and diameter and length of delivery pipe are 4cm and 20m respectively. Find the maximum speed of the pump to avoid the separation wither in suction pipe of delivery pipe. Take atmospheric pressure head = 10.33m of water and separation takes place at 80 kN/m2 below atmospheric pressure.										(12)

(OR)

25.	a. 	The diameters of a centrifugal pump, which is discharging 0.03m3/s of water against a total

head of 20m is 0.4m. The pump is running at 1500 rpm. Find the head, discharge and ratio of powers of a geometrically similar pump of diameter 0.25m when it is running at

3000 rpm.																	(10)

b 	Define Manometric, Mechanical and Overall Efficiencies for a centrifugal pump and relationship between them.													(5)