Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF SOILS							               Time : 3 hours

Subject Code: 	CE247			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Liquid and plastic limit exist in ---------------------- soil.

2.	The ratio of the volume of water present in a given soil mass to the total volume of voids is known as ------------------------

3.	Darcy’s law is applicable to seepage if a soil is -------------

4.	A coarse –grained soil has a void ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is ---------

5.	A soil not fully consolidated under the existing over burden pressure is called-------

6.	The ratio of settlement at any time “t” to the final settlement , is known as -------

7.	----------- is a curve or contour connecting all points below the ground surface of equal vertical pressure.

8.	If the footing is flexible, the distribution of contact pressure is --------

9.	Vane shear test is done only for clay soil. Say true or false

10.	The two basic types of finite slope failure are --------- and -----------

PART – B  (5 x 3 = 15 MARKS)

11.	Enumerate and define Atterberg’s limits.

12.	Explain quick sand phenomenon.

13.	Explain optimum moisture content and its significance in field compaction.

14.	Differentiate immediate and consolidation settlement.

15.	Brief the different kinds of slope failure.

PART – C  (5 x 15 = 75 MARKS)

16.	The soil sample has a diameter of 38mm and height 76mm. its wet weight is 1.15N. upon drying its weight reduced to 0.5N. Gs = 2.7. In the wet state what was the degree of saturation, void ratio, porosity and the water content of the soil sample. Comment on the value of the same.

(OR)

17.	A soil has a plastic limit of 25% and the plasticity index of 30. If the natural water content of the soil is 34%, what is liquidity index and what is the consistency index? How do you describe the consistency?

18.	In a falling head permeability test, head causing flow was initially 50cm and it drops 2cm in 5minutes. How much time required for the head to fall to 25cm?

(OR)

19.	Water flows at the rate of 0.09 ml/s in an upward direction through a sand sample with a coefficient of permeability of 2.7 x 10-3 m/s. The thickness of the sample is 120mm and area of cross section is 5400mm2. Taking the saturated unit weight of the sand as 18.9 kN /m3, determine the effective pressure at the middle and bottom of the sample.

[P.T.O]

20.	A clay soil, tested in a consolidometer, showed a decrease in void ratio from 1.20 to 1.10 when the pressure was increased from 0.25 to 0.50kgf /cm2. Calculate the coefficient of compressibility (av) and the coefficient of volume compressibility (mv). If the coefficient of consolidation (Cv) determined in the test for the given stress increment was 10 m2 /year, calculate the coefficient of permeability in cm/s.

If the sample tested at the site was taken from a clay layer 3.0m in thickness, determine the consolidation settlement resulting from given stress increment.

(OR)

21.	There is a bed of compressible clay of 4m thickness with pervious sand on the top and impervious rock at the bottom. In a consolidation test on an undisturbed specimen of clay from this deposit 90% settlement was reached in 4hrs. The specimen was 20mm thick. Estimate the time in years for the building founded over this deposit to reach 90% of its final settlement.

22.	A concentrated load of 22.5kN acts on a homogeneous soil mass of large extent. Find the stress intensity at a depth of 15m and i) directly under the load, and ii) at a horizontal distance of 7.5m. Use Boussinesq’s equation.

(OR)

23.	An elevated structure with a total weight of 10000kN is supported on a tower with 4 legs. The legs rest on pier located at the corner of a square 6m on a side. What is the vertical stress increment due to this loading at a point 7m beneath the centre of the structure?

24.	Calculate the potential shear strength on a horizontal plane at a depth of 3m below the surface in a formation of cohesion less soil when the water table is at a depth of 3.5m. The degree of saturation may be taken as 0.5on the average. Void ratio = 0.50, grain specific gravity = 2.70, angle of internal friction = 30 degree. What will be the modified value of shear strength if the water table reaches the ground surface?

(OR)

25.	A cut 9m deep is to be made in clay with a unit weight of 18 kN /m3 and a cohesion of        	  	27 kN /m2. A hard stratum exists at a depth of 18m below the ground surface. Determine from 	Taylor’s chart if a 30 degree slope is safe. If a factor of safety 1.50 is desired, what is a safe 	angle of slope? Take for D = 2, β = 30º then   N =0.172 and for N = 0.11 and D = 2 then β = 8º.