Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FOUNDATION ENGINEERING								Time : 3 hours

Subject Code:	CE248			       						                	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the criterion for depth of soil exploration for a raft of size B x L.

2.	What is the difference between a ‘continuous footing’ and a ‘combined footing’?

3.	State the values of Nc, Nγ and Nq    at   Φ =0.

4.	If you underestimate Φ, how does it affect the design of a footing ?

5.	How are c and  φ affected by submergence ?

6.	Why is the displacement necessary for mobilizing passive pressure much higher than that is

required for mobilizing active pressure ?

7.	What do you understand by ‘group efficiency’ of a pile group ?

8.	What is a ‘compensated raft’ ?

9.	What is the difference between ‘caissons’ and ‘cofferdams’?

10.	Why is the cutting edge of a well foundation made wider than the steining ?

PART – B  (5 x 3 = 15 MARKS)

11.	How is the eccentricity of load considered in determining the bearing capacity of a footing ?

12.	What is the difference in the approach between Rankine’s and Coulomb’s earth pressure

theories ?

13.	Find the value of   υ (Poisson’s ratio) which gives the same value of K0 at Φ = 300.

14.	Using standard values for unit weights and the angle of internal friction, show that water

pressure is twice the active earth pressure due to dry soil and thrice the active earth pressure

due to submerged soil.

15.	Write the expressions for shear strength between two parts of the same soil and between

soil and a wall at their interface.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the test set-up and explain the ‘Standard Penetration Test’.  Explain the corrections to

be applied to the results.

(OR)

17.   What are the steps performed by the foundation engineer to arrive at the most appropriate

foundation for given structure at a given site ?

18.	a.	What are the factors influencing Nc according to Skempton?  Plot the variation of  Nc

with these factors.                                                                                                               (6)

b.	A footing of size 2 x 2 m carries a central vertical load of  600 kN.  How should the size

of the footing be altered if it is called upon to carry also moments of 100 kNm in one

direction and 60 kNm in the other direction ?                                                                       (9)

(OR)

[P.T.O]

19.   Plot the load-settlement data given below obtained from a plate bearing test conducted

using a plate of size 0.75 x 0.75 m.  Determine (1) the bearing capacity (in kN/m2) and

(2) the settlement of a footing of size 1.5 x 1.5 m at a load of  280 kN.

.                        			Load (kN)            		50       100     200      300     400     500     600

Settlement (mm)  		1.5       2.0      4.0       7.5      12.5   20.0    40.0

20.  a.     Derive an expression for the coefficient of active earth pressure in the case of a   Φ -soil. (6)

b.     For the same soil, drawing full Mohr’s circles, show the pairs of potential failure planes

in both the active and passive states.                                                                                    (5)

c.     State the importance of drainage of backfills behind retaining walls.                                  (4)

(OR)

21.	For the data given below, determine the magnitude, direction and point of application of the 	active thrust by Poncelet’s graphical method.  (Draw on one page of your answer book.)  Also 	mark the figure of rupture and measure its inclination from the horizontal.

Data:  Height of wall 4 m, Inclination of the soil side of the wall with the horizontal – 800

Angle of wall friction – 150  , Inclination of the ground line – 100  , Angle of  internal

friction of the cohesionless backfill – 320   , Unit weight of backfill – 18 kN/m3

22.  a.     Show by a numerical example that, for the same volume of concrete, a number of piles of

smaller diameter is more efficient than a single pile of larger diameter in transferring

load by friction.                                                                                                                     (5)

b.     Write a note on ‘negative skin friction’ and how it can be minimized.                                (5)

c.     Discuss the effectiveness of the bulb of an underreamed pile established in an expansive

soil.                                                                                                                                        (5)

(OR)

23.   For a group of friction piles in a cohesive soil, as per the following data, determine the group

capacity, the sum of the individual capacity and the group efficiency.                                     (10)

If square piles of equivalent cross section are proposed to be used in place of the circular piles,

find how the results get modified.                                                                                               (5)

Data:  Pile dia – 320 mm, Pile length – 12 m, No. of piles in the group – 12 (3 rows, 4 columns)

Pile spacing – 960 mm along the row, 1m along the column), cohesion –

96 kN/m2   , Adhesion factor – 0.55, Factor of safety – 3.

24.	a.	What are the different types of combined footings ?  Answer with suitable sketches.         (6)

b.	For the data given, determine the plan dimensions of a trapezoidal type of combined

footing.

Data:  Column loads: Col A – 1050 kN, Col. B – 1340 kN. Col. spacing – 5.8 m, Footing

not to project more than 0.4 m from the centre of Col. A and 0.6 m from the centre

Of Col. B, Allowable soil pressure – 160 kN/m2                                                     (9)

(OR)

25.    a.   Draw a neat sketch showing the different parts of a well foundation.                                 (9)

b.   Explain the steps involved in the sinking of the well in an artificial island.                       (6)