Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	REINFORCED CONCRETE STRUCTURES - I				Time : 3 hours

Subject Code:	CE249			       						                	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the ultimate limit in collapse.

2.	What is the serviceability limit state.

3.	What is the expression for cracking moment.

4.	What is a tension failure in beam.

5.	Give the limiting depth of neutral axis for Fe 250 & for Fe 415.

6.	What is the expression for accidental eccentricity in column.

7.	Write down the expression for the ultimate axial load capacity of a shout column.

8.	While considering the design of an isolated column footings, critical section for bending moment and two way shear.

9.	The effective width of T – beam is given by.

10.	Maximum shear strength of concrete for m20 grade concrete.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain limit states.

12.	Derive an expression for the ultimate flexural strength of a beam section.

13.	What are modes of failure of a column, when centrally loaded.

14.	What are different cases to be considered for the position of neutral axis for T- beam design.

15.	What are different modes of cracking to be noted for reinforced concrete beam under shear.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the area of reinforced required for a singly reinforced section having a breadth of     300 mm and effective depth of 675 mm to support a factored moment of 185 kNm. Adopt                fck = 20N/mm2   and fy=415 N/mm2

(OR)

17.	Design the minimum effective depth required and area of tension reinforcement for a rectangular beam having a width of 300 mm to resist an ultimate moment of 200 kNm, using M-20 grade concrete and Fe 415 grade steel.

18.	Determine the ultimate flexural strength of the T-beam having the following section properties.

Width of flange (bf) 	 	= 800mm.

Depth of flange (Df)	 	= 150mm

Width of r/b (bw) 			 = 300 mm

Effective depth (d) 		 = 420mm

Area of steel 				 = 1470 mm2

fck = 25N/mm2  and fy		 = 415 N/mm2

(OR)

[P.T.O]

19.	Determine the ultimate moment of distance of a doubly reinforced beam of rectangular section having a width of 300 mm and reinforced with 5 nos- 25mmφ at an effective depth of 600 mm. The compression steel consists of 2nos – 25mm φ at and effective cover of 60mm.        fck = 20N/mm2  and fy = 415N/mm2.

20.	Design the reinforcements in a column of size 400mm by 600mm subjected to an axial (working) load of 2000 kN. The column has an unsupported length of 3m and is braced against sideway in both directions Adopt M-20 grade concrete and Fe 415 grade steel.

(OR)

21.	Determine the reinforced of grade Fe 415 steel required for a square column of size 400 x 400 mm subjected to a ultimate direct load and ultimate moment of 2000 kN and 250 kNM, respectively. When reinforced is to be placed on two sides (parallel) with a nominal cover of 30mm use fck = 20 N/mm2 and fy = 415N/mm2. Use column interaction curve.

22.	A reinforced concrete beam of rectangular section 300 mm wide by 650 mm overall depth is reinforced with 4nos – 32 mm φ at an effective depth of 600 mm use M-20 concrete and Fe 415 grade steel. estimate moment of resistance of section.

(OR)

23.	A reinforced concrete beam of rectangular section is required to resist a service moment of      120 kNm. Design suitable dimensions and reinforcements of the beam assuming M20 grade concrete and Fe 415 grade steel.

24.	Design a two-way slab 4 m x 6m for residential floor to suit the following requirements edge conditions: Slab simply supported without any provision for torsion at corners. Use M – 20 grade concrete and Fe415 grade steel.

Live load = 2kN/m2

Floor finish = 60 kN/m2

(OR)

25.	Design a suitable R.C.C column to support and axial load of 800 kN. Adopt M20 and Fe 415 grade steel.