Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DESIGN AND DRAWING (RCC AND STEEL)		               Time : 4 hours

Subject Code: 	CE251			         						                Maximum Marks: 100

(IS 456, 3370 IRC codes are permitted)

#### Answer ALL questions

PART – A  (1 x 55 = 55 MARKS)

1.	Design a counterfort retaining wall to retain sand for 6.5m above the general ground level. The earth is submerged at an angle of 15о The angle of repose of earth is 30 о and its density is 16000N/m3  The bearing capacity of soil is 200 kN/m2 Coefficient of friction = 0.6.

a. 	Draw the section through wall between two counterforts

b.	Draw the section through counterforts

Assume allowable stresses as σcbc = 7 N/mm2  σcbc = 140 N/mm2  for bars above 20mm

Q = 1.22 j =0.867

(OR)

2. 	Design the rectangular tank placed on the ground having a capacity of 80,000 litres. Height of the tank is 3.5m with a free board of 150mm. Assume L/B =1.5. The bearing capacity of soil is 200 kN/m2 Assume allowable stresses as σcbc = 7 N/mm2  σcbc = 140 N/mm2  Use M20 concrete and grade 1 mild steel.

a.	Draw the section along the length.

b.	Draw the section along the width and horizontal cross section.

PART – B  (1 x 45 = 45 MARKS)

3. 	A built up steel column comprising two ISWB 400 RSJ sections with their webs spaced at 325mm and connected by 10mm thick battens transmits an axial load of 2000 kN.  The safe bearing capacity of soil is 300KN/m2 The safe permissible stress on the concrete bed = 4N/mm2  Design a suitable foundation for the column adopting a slab base and sketch the details of the foundation.

(OR)

4. 	Design a tubular steel truss to suit the following data.

Span of truss   	=	16m

Type of truss      	= 	fink truss

Roof cover         	= 	GC sheeting

Spacing of roof trusses   	= 	4.5m

Wind pressure                	= 	1 kN/m2

Steel tubular sections conforming to IS 1161 -1976 are available for use

Draw the elevation of roof truss.