Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DESIGN OF STEEL STRUCTURES					               Time : 3 hours

Subject Code: 	CE252			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The permissible stress in axial tension on the net effective area of the section shall not exceed ____________

2.	Effective length of column effectively held in position and restrained in direction at both the ends, is _____________

3.	Web crippling occurs at the point where ___________ load acts.

4.	In a built up beam actual bending compressive stress fcb is given by _______

5.	The economic depth of web in welded plate girder is given by __________

6.	The welded plate girder weighs less than the riveted plate girders. The saving may vary from 	_____________ percent depending on the governing specifications.

7.	Width of industrial building units with gabled roofs may vary from ___________

8.	The member of a roof truss which supports purlins is called ____________

9	The buckling in which the compression member can bend and twist simultaneously is called __________.

10.	Give the shapes used for roof decks, floors, etc. in light gauge members.

PART – B  (5 x 3 = 15 MARKS)

11.	Under what circumstances lacings are provided?

12.	Define slender beam.

13.	Show the elements of plate girder with neat sketch.

14.	Define purlin.

15.	What is meant by effective design width?

PART – C  (5 x 15 = 75 MARKS)

16.	Design a single angle section for a tension member of a roof truss to carry a pull of 100kN. The member is subjected to possible reversal of stress due to action of wind. The length of the member from center to center of intersection is 3.50 m.

(OR)

17.	A column section HB 250 @ 0.510 kN /m carries an axial load of 6000 kN. Design a slab base for the column. The allowable bearing pressure on concrete is 4 N / sq.mm. The allowable bending stress in the slab base is 185 N/sq.mm.

18.	A compound beam is built up with channel ISLC 200 @ 20.6 kg/m attached to the top flange of ISMB 450 @ 72.4 kg/m. Calculate the maximum central load the beam can carry over a span of 7.6m. When the bending stress is limited to 165 MPa. Neglect the self weight of the beam.

(OR)

19.	Design a suitable section for a beam girder carrying udl of 65 kN/m including self weight over an effective span of 16m.

20.	A welded plate girder consists of the following elements:

Top Flange – 290 x 22 mm, Bottom flange – 215 x 22 mm web – 900 x 8 mm.The beam is subjected to 28 kN / m UDL. Determine the bending stresses if L = 16m

(OR)

21.	A MB 500 @ 0.869 kN / m transmits an end reaction of 130 kN and an end moment of

120 kN-m to the flange of a column HB 250 @ 0.510 kN/m. Design the welded connection.

22.	Design an angle iron purlin for a trussed roof from the following data: Span of roof truss = 12 m, Spacing of roof truss = 5m, Spacing of purlins along the slope of roof truss = 1.2 m, Slope of roof truss = 1 Vertical to 2 Horizontal, Wind load on roof surface normal to roof = 1.04 kN/sq.m, Vertical load from roof sheeting etc., = 0.2 kN / sq.m.

(OR)

23.	Explain the step by step procedure followed in the design of Gantry Girder.

24.	Find the allowable load for the rectangular tubular column section shown in Fig.1. The effective length of column is 3.6 m. Take fy = 235 N / Sq.mm

5

190mm

5

2mmM

5

Fig. 1

110

5

(OR)

25.	Determine the allowable load per meter run on a beam with hat section shown in fig.2. The effective span of beam is 3m. Steel has a yield point of 235 N / sq.mm and

E = 2 x 105 N / sq.mm.

120 mm

2

5

5

Fig. 2