Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	SANITARY ENGINEERING								Time : 3 hours

Subject Code:	CE253			       						                	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Find the dry weather flow ( DWF) of a city having the following particulars Population 20 lakhs water supply rate 130 LPCD assume 60% of the water supplied reaches as sewage to the sewer.

a.	1.806 m3/ sec  			b.	1.406 m3/sec

2.	The most suitable section of sea ware in separate sewage system is ________

3.	The floating and settable solids from the waste water can be removed by

a.	Primary Treatment   		 b.	 Screening

4.	The particles which do not change in size shape (or) mass during settling are known as ------------

5.	Algal bacterial symbiosis will take place while treatment the waste water in

a.	Septic tank  				 b.	Oxidation pond

6.	The example for the aerobic attached growth process of treatment of waste water is _________

7.	The biogas comprises of

a.	CH4 and CO2		 b.	CH4 and O2

8.	The treated sludge could be used as -----------

9.	The main purpose of providing Traps is

a.	To prevent the entry of foul gas in the house

b.	Easy cleaning and maintenance

10.	Chlorine is used in the treatment of sewage to remove ___________

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the terms Sewage, Sullage, Refuse

12.	State the basic principle of sedimentation

13.	Estimate the quality of wet weather flow of a city having the area as 3000 hectare intensity of 	rain fall is 1.5 cm/ hr and the average impermeability factor is 0.6

14.	Draw the neat flow diagram of ASP and indicate various operations involved in it.

15.	List out any five water conservation techniques.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	List out the sewer appurtenances and explain any two with neat sketches       	(2+4+4)

b.	Calculate 1 day 370C BOD of sewage sample whose 5 day 200C BOD

is 100 mg /l. Assume KD at 200C as 0.1. day -1 Take KD(T)= KD20{1.047}T-20       		(5)

(OR)

17.	a.	A combined sewer of a circular section is to be laid to serve a particular area.   		(10)

Calculate the size of this sewer from the following data

Area to be calculated 		 	     		= 120 hectares

Population 				    	 		= 1, 00,000

Maximum permissible flow velocity      	 = 3 m/ sec

Time of entry for storm water 		 	= 10 mts

[P.T.O]

Time of flow in channel				= 20 mts

Per capita water supply				= 250 liters/ day /person

Co efficient of run- off for the area 		= 0.45

Hourly, Maximum rainfall for the area at

the design frequency 				= 5 cm

Take 80% of water supply comes out as sewage.

Maximum sewage discharge = 3 times the average discharge.

Assume sewer is running full and maximum velocity of 3 mts /sec at maximum flow

b.	Compare conservancy and water carriage system of collection and disposal of the refuse from urban area 																(5)

18.	a.	Design suitable rectangular sedimentation tank for treating the sewage from a city, provided

with an assured public water supply system, with a maximum daily demand of 12 million liters per day. Assume detention period as two hours and velocity of   flow 0.3 meters / minute in the tank. Take 80% of water supply comes out as sewage.								 (10)

b.	List out the objectives of the sewage treatment 									 (5)

(OR)

19.	a.	Describe the grit chamber with special reference to their purpose location, nature of grit and 		velocity control devices used in grit chamber	.									 (8)

b.	Discuss the broad classifications of the sewage treatment process 					 (7)

20.	a.	Compare the salient features of standard rate trickling filters with High rate

Trickling Filters  																(10)

b.	Write short notes on Rural sanitation 										  	(5)

(OR)

21.	a.	Design a septic tank with the following data 										  (9)

No of people 						= 300

Sewage /capita/day					= 120 liters

De-slugging period					= 3 year

Length: width 						= 4: 1

Peculation capacity of filter media in soak well 	= 1300 L/m3 /day

Depth of soak well 					= 2 mts

Rate of sludge deposit 				= 30 L/ capita / year

Depth of septic tank					= 2 mts

b.	Differentiate aerobic & anaerobic biological process and attached growth & suspended 			growth process														  		      (6)

22.	a.	What is mean by Environmental pollution? Describe what happens when untreated sewage

from a town is discharged into a near by stream		         						     (7)

b.	Write short notes on various methods of Sludge disposal  		     					     ( 8)

(OR)

23.	a.	Explain the sludge digestion process and the useful end product					     (9)

b.	What do you mean by broad irrigation and effluent irrigation 						     (6)

24.	a.	List out the general principle of house drainage system   								(8)

b.	Explain the different methods of Rain water harvesting 								(7)

(OR)

25.	a.	List out the various sanitary fillings and explain their uses with neat sketch                       (7)

b.	Write short notes on:

i.	Maintenance of house drainage system	ii.	Need for Recycling of waste water.	  (4+4)