Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	STRUCTURAL ANALYSIS – I								Time : 3 hours

Subject Code:	CE254			       						                	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the principle of Virtual work?

2.	State whether the truss is statically determinate or not?

3.	When two wheel loads w1 and w2 cross a simply supported span and if intensity of w2 > w1. What is the position of absolute maximum bending moment?

4.	List the uses of influence line diagram?

5.	In two hinged parabolic arch subjected to udl of intensity w kN/m , by applying Castigliano’s  second theorem , the horizontal thrust is given by the expression ___________

6.	State whether the three-hinged arch is a determinate or not?

7.	Define Tension Coefficient of any member?

8.	In suspension bridges, the maximum dip of the cable varies form ___________ to _____________

9.	The method of consistent deformation falls in the category of ___________

10.	Which graphical method is employed to determine the deflections of statically determinate truss structure?

PART – B  (5 x 3 = 15 MARKS)

11.	State Betti’s Maxwell law?

12.	State Muller Breslau Principle?

13.	What is the position and magnitude of positive and negative bending moment in a three hinged parabolic arch subjected to point load rolling over the arch?

14.	A suspension cable having supports at same level has span of 30m and maximum dip of 3m. The cable is loaded with udl of 10 kN/m throughout the length. Find the maximum tension in the cable?

15.	Why compatibility conditions are required?

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	The horizontal displacement at support D is to be determined for the frame shown in figure 1      E = 200 GPa. I = 300 x 106 mm4. Analyze the frame by virtual work method.

2I

B

C

2m

50 KN

I

I

2m

Fig.1

A

D

6m

(OR)

17.	Using Castigliano’s second theorem determine the vertical and horizontal deflection components of joint C of the truss shown in figure 2. E = 200 GPa. Cross sectional area of each bar is  100 mm2

100KN

C

Fig.2

300

600

A

B

2m

18.	Two point loads of 100 kN and 200 kN spaced 3m apart cross a girder of span 15m from left to right with 100kN load leading. Draw the influence line diagram for shear force and bending moment at a section D, 6m from left hand support. Also find the absolute maximum bending moment.

(OR)

19.	Four equal loads of 150 kN each equally spaced at 2m apart followed by udl of 60 kN/m at a distance of 1.5m from last 150 kN load cross a girder of 20 m span from left to right. Using influence line calculate shear force and bending moment at section 8 m from left hand support when leading 150 kN load is 5m from left hand support

20.	A symmetrical three hinged parabolic arch of span 40 m and rise 8m carries udl of intensity       30 kN/m over the left half of the span. Hinges are provided at the supports and at the centre of the arch. Calculate the reactions at the support s. Also calculate the bending moment, radial shear and normal thrust at a distance of 10 m from left support.

(OR)

21.	A three hinged circular arch of span 16 m and rise 4m is subjected to two point loads of 100 kN and 80 kN at left and right quarter span points respectively. Find the reactions at the supports. Find also the radial shear, bending moment and normal thrust at 6 m from left support.

22.	Using method of tension coefficients, analyze the space truss as shown in figure 3 Find the forces in the members of the truss.

75 KN

D

50 KN

5m

A

B

C

3m

3m

f

Fig.3

Elevation

B

2m

D

50 KN

2m

A

C

6m

plan

(OR)

23.	A Supension bridge of 250 m span has two numbers of three hinged stiffening girders supported cables with central dip of 25m. If four point loads of 150 kN each are placed at the centre line of roadway at 20m , 30m , 40 m and 50m from left hand hinge. Find the shear force and bending moment in each girder at 62.5m from each end. Calculate also the maximum tension in the cable.

24.	A portal frame fixed at ends is shown in figure 4. Calculate the end reactions using method of consistent deformation

50KN

B

C

3m

3m

3 m

5m

2I

I

25 KN

Fig.4

I

45 m

D

A

(OR)

25.	A continuous beam is fixed at A as shown in figure 20.b. Analyze the beam using theorem of three moments. EI is constant throughout. Also draw the S.F and B.M diagram

2 t

1 t/m

0.5 t/m

1 t/m

A

B

C

D

10 m

15 m

10 m

2m

Fig.5