Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DESIGN AND DRAWING (IRRIGATION AND ENVIRONMENTAL

ENGINEERING)											Time : 4 hours

Subject Code:	CE256			         						                Maximum Marks: 100

Assume suitably any missing data

#### Answer ALL questions

PART – A  (1 x 55 = 55 MARKS)

1.	Design a tank sluice with a tower head using the following hydraulic particulars:			(20)

Ayacut				100 hectares

Duty 				800 hectares / cumec

Top width of bank 	1.8  m

Front slope 			1.5 : 1

Rear slope 			2 : 1

T.B.L      			20.00 m

M.W.L    			18.70 m

F.T.C       			18.1 m

Highest land level 	14.40 m

Lowest land level 	12.00 m

Well diameter		1.20 m

Draw to a suitable scale the following views:

a.	Longitudinal section                                                         							(15)

b.	Half plan at top and half at foundation                               							(20)

(OR)

2.  	Design a tank surplus weir with the following particulars:								(20)

Area of combined catchment		=	50 Sq.km

Area of intercepted catchment		=	40 Sq.km.

Full tank level					=	110.00 m

Maximum water level				= 	111.20 m

Top of bund level					=	112.45 m

Ground level						=	109.70 m

Top of formation level			=	108.00 m

Bed level of surplus channel		=	108.40 m

Front slope						=	1.5 : 1

Rear slope						=	2:1

Draw to a suitable scale the following views:

a.	Longitudinal section						    									(15)

b.	Half plan at top and half at bottom.  				     							(20)

[P.T.O]

PART – B  (1 x 45 = 45 MARKS)

3.	a.	Design a plain rectangular sedimentation tank for a water works which supplies 1.5 million

liters of water per day to a city. The sedimentation period is 6 hours. The velocity of flow is 14cm per minute. The depth of the water in the tank is 4m. Assume an allowance for sludge as 85 cm. Provide baffles at suitable intervals.                                                         	(15)

b.	Draw to the suitable scale the plan and the cross section of the tank.      	 		(15+15)

(OR)

4.	A tank of 1,50,000 population is to be provided with a treatment plant using trickling filter. The per capita contribution of sewage is 90 litres/day.

Design the primary secondary settling tanks and the trickling filter.               				(15)

Draw to a suitable scale the following:

a.	Section of trickling filter                                                                       				(15)

b.	Plan of the Trickling filter                                                                     				(15)