Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF FLUIDS									Time : 3 hours

Subject Code:	CE277			       						                       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between absolute pressure and gauge pressure

2.	Define metacentric height

3.	What is meant by streak line

4.	What is flow net

5.	What is the use of pitot tube

6.	Write the expression for discharge over a trapezoidal weir.

7.	What is meant by laminar flow

8.	What is the use of moody’s diagram

9.	What is the dimension of kinematic viscosity

10.	What is mean by dimensional homogeneity

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by centre of pressure.

12.	The diameters of a pipe at two sections 1-1 and 2-2 are 200mm and 300mm respectively. If the velocity of water flowing through the pipe at section 1-1 is 4 m/s. Find : i.	 Discharge through the pipe and    ii.	 velocity of water at section 2-2

13.	What are the advantages of a triangular notch over a rectangular notch ?

14.	List the types of energy loss in flow through pipes

15.	What are the similarities that must be ensured between the model and the prototype

PART – C  (5 x 15 = 75 MARKS)

16.	The space between two square flat parallel plates is filled with oil. Each side of the plate is 720 mm. The thickness of the oil film is 15mm. The upper plate, which moves at 3m/s requires a force of 120 N to maintain the speed. Determine : a.	the dynamic viscosity of the oil

b. the kinematic viscosity of oil if the specific gravity of the oil is 0.95

(OR)

17.	Fig 1 shows a U-tube differential manometer connecting two pressure pipes at A and B. The pipe A contains a liquid of sp.gravity 1.6 under a pressure of    110 KN/m2. The pipe B contains oil of sp.gravity 0.8 under a pressure of 200 KN/m2. Find the difference of pressure measured by mercury as fluid filling U-tube.

[P.T.O]

18. The velocity components in a fluid flow are given by :

u = 2xy,  v = a2+x2 – y2

i) show that the flow is possible

ii) Derive the relative stream function.

(or)

19. A pipe 200m long slopes down at 1 in 100 and  tapers from 600mm diameter at the higher end to 300 mm diameter at the lower end and carries 100 litres/sec of oil (sp.gr of 0.8). If the pressure gauge at the higher end reads 60 KN/m2, determine :

i) Velocities at the two ends

ii) Pressure at the lower end.

Neglect all losses.

20. Derive an expression for maximum discharge over a broad crested wier

(or)

21. A horizontal venturimeter with inlet diameter 200mm and throat diameter 100 mm is used to measure the flow of water. The pressure at inlet is 0.18 N/mm2 and the vacuum pressure at the throat is 280 mm of mercury. Find the rate of flow. The value of Cd may be taken as 0.98.

22. Oil of specific gravity 0.82 is pumped through a horizontal pipe line 150mm in diameter and 3 km long at the rate of 0.015 m3/sec. The pump has an efficiency of 68% and requires 7.5 KW to pump the oil.

i) What is the dynamic viscosity of the oil

ii) Is the flow laminar.

(or)

23. In a pipe of 300 mm diameter and 800 m length an oil of specific gravity 0.8 is flowing at the rate of 0.45 m3/s. Find :

i) Head lost due to friction  and

ii) Power required to maintain the flow.

24. Describe Buckingham’s method or -theorem to formulate a dimensionally homogeneous equation between the various physical quantities effecting a certain phenomenon.

(or)

25. Prove that velocity through an orifice can be expressed as:

where, H = head causing flow

D = Diameter of orifice

 = Co-efficient of viscosity

 = mass density

 = surface tension

A

B

2.6 m

1 m

X

X

Fig 1

•  A

18.	The velocity components in a fluid flow are given by :

u = 2xy,  v = a2+x2 – y2

a.	show that the flow is possible	b.	Derive the relative stream function.

(OR)

19.	A pipe 200m long slopes down at 1 in 100 and  tapers from 600mm diameter at the higher end to 300 mm diameter at the lower end and carries 100 litres/sec of oil (sp.gr of 0.8). If the pressure gauge at the higher end reads 60 KN/m2, determine :

a.	Velocities at the two ends

b.	Pressure at the lower end.

Neglect all losses.

20.	Derive an expression for maximum discharge over a broad crested weir

(OR)

21.	A horizontal venturimeter with inlet diameter 200mm and throat diameter 100 mm is used to measure the flow of water. The pressure at inlet is 0.18 N/mm2 and the vacuum pressure at the throat is 280 mm of mercury. Find the rate of flow. The value of Cd may be taken as 0.98.

22.	Oil of specific gravity 0.82 is pumped through a horizontal pipe line 150mm in diameter and 3 km long at the rate of 0.015 m3/sec. The pump has an efficiency of 68% and requires 7.5 KW to pump the oil.

a.	What is the dynamic viscosity of the oil 		b.	Is the flow laminar.

(OR)

23.	In a pipe of 300 mm diameter and 800 m length an oil of specific gravity 0.8 is flowing at the rate of 0.45 m3/s. Find :

a.	Head lost due to friction  and

b.	Power required to maintain the flow.

24. Describe Buckingham’s method or -theorem to formulate a dimensionally homogeneous

equation between the various physical quantities effecting a certain phenomenon.

(OR)

25.	Prove that velocity through an orifice can be expressed as:

V=2gH  φDH,μρVH,σρV2H

where, H = head causing flow

D = Diameter of orifice

 = Co-efficient of viscosity

 = mass density

 = surface tension