Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	IRRIGATION ENGINEERING						               Time : 3 hours

Subject Code: 	CE278			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Irrigation is broadly classified into ________and _______irrigation.

2.	Flow duty is expressed as __________.

3.	Define an Inundation canal.

4.	Define Sodium Absorption Ratio.

5.	What is  Aquifuge?

6.	Define Leaching.

7.	Mention the relation between critical velocity to the depth of flow.

8.	Lacey’s Theory is not applicable to _______regime channels.

9.	A gravity Dam requires _________ maintenance.

10.	Low water training is done to maintain ______________.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the delta for a crop when its duty is 864 hectares / m3 on a field, the base period for the crop is 120 days.

12.	What are the classifications of Sprinkler irrigation?

13.	Mention the types of Tube wells.

14.	Classify the canals according to canal alignment.

15.	Mention the disadvantages of Arch Dams.

PART – C  (5 x 15 = 75 MARKS)

16.	Mention the advantages and ill effects of irrigation.

(OR)

17.	a.	Explain the direct measurement of consumptive use of water in the field.			(8)

b.	A distributory is to be designed for irrigating 5000 hectares in Rabi season and 2000 hectares in Kharif season. If water requirements for Rabi and Kharif season crops are 12cm and 22cm and water applied for the crops for 2.5 weeks, and 4 weeks respectively, determine the discharge of the distributory.										(7)

18.	a.	Explain the functioning of a Weighing bucket type automatic rain gauge with a sketch.	(8)

b.	Explain the methods available for calculating the average rainfall over a basin.		(7)

(OR)

19.	a.	What are the limitations of sprinkler irrigation method.							(8)

b.	What are the various types of impurities which make the water unfit for irrigation.	(7)

20.	a.	Explain the Recuperation Test for measuring the yield from an open well.			(8)

b.	During a Recuperation test, the water level in the well was depressed on pumping by 2.50

metres and is recuperated by an amount of 1.60 metres in 70 minutes. Determine the yield from a well of 3m diameter under a depression head of 3.50 metres. 				(7)

[P.T.O]

(OR)

21.	a.	A 30cm diameter well penetrates 25 metre below the static water table. After 24 hours of

pumping at 5400 litre / minute the water level in the test well at 90 m is lowered by 0.53 metre and in a well 30 metre away, the draw down is 1.11 m, what is Transmissibility of aquifer.																		(8)

b.	State Dupuit’s assumptions in the theory of flow for an unconfined aquifer.		       (7)

22.	a.	What are channel losses?														(3)

b.	Explain the advantages of canal lining.											(12)

(OR)

23.	By the application of Lacey’s Theory design a canal for the following data:

Discharge Q 			=	30 m3/s

Lacey’s Silt factor	=	1.00

Canal side slope		= ½ :1

Find also the longitudinal slope.

24.	a.	Mention the types of river training works.										(8)

b.	Enumerate the physical factors governing the selection of type of a dam.			(7)

(OR)

25.	a.	What are the functions of a Head regulator and a Cross regulator.					(8)

b.	What is cross drainage structure and mention its types.							(7)