Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	WATER SUPPLY AND SANITARY ENGINEERING			Time : 3 hours

Subject Code:	CE279			       						                	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by the term “percapita demand”?

2.	What are the various methods of population forecasting?

3.	What are the characteristics of ground water?

4.	What is Electro-Katadyn process?

5.	What are the various methods of analysis of distribution system?

6.	List out the house water connection appurtenances.

7.	What is the principle behind sedimentation?

8.	Define sewage.

9.	What is F/M ratio?

10.	What is oxidation pond?

PART – B  (5 x 3 = 15 MARKS)

11.	Find the equivalent length of 45 cm dia. pipe for the network shown below. Adopt Hazen

William Formula.

12.	Write short notes on “Zeolite process”.

13.	What are the requirements of good distribution system?

14.	A gritchamber is designed to remove particles with a diameter of 0.2mm, specific gravity 2.65.Settling velocity for these particles has been found to range from 0.016 to 0.022m/sec, depending on their shape factor. A flow through velocity of 0.3 m/sec will be maintained by proportioning weir. Determine the channel dimensions for a maximum wastewater flow of 10,000 cu.m/day.

15.	What are the design considerations adopted in the design of septic tank?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A water supply scheme has to be designed for a city having a population of 2 lakhs.

Estimate the important kinds of drafts which may be required to be recorded for an average water consumption of 180 lpcd. Also record the required capacities of the major components of the proposed water works system for the city using a river as the source of supply. Assume suitable data wherever required.   	                                                                     (12)

b.	Write the drinking water quality standards for

i.	Colour	ii.	Chlorides 	iii.	Turbidity 	iv.	Manganese 		v.	pH                          (vi) Flourides															            (3)

(OR)

17.	a.	In two periods, each of twenty years, a city population has grown from 30,000 to 1, 72,000

and  to  2, 92,000. Find

i.	Saturation Population		ii.	Coefficients of logistic curve	iii.	Equation of logistic curve	iv.	Expected population in the next 20 years.					           (10)

b.	Write short notes on rain water harvesting 						                                (5)

18.	a.	Distinguish between slow sand and rapid sand filters.								    (10)

b.	Explain the operational troubles in rapid gravity filters.							      (5)

(OR)

19.	a.	What is disinfection? Explain in detail the minor methods of disinfection.  			     (10)

b.	Explain “Nalgonda Technique” for flouride removal.									(5)

20.	Find the equivalent length of 30 cm dia. pipe for the network shown in figure. Adopt     HazenWilliam formula.

(OR)

21.	a.	Explain in detail the methods of leakage detection in distribution pipes.				   (7)

b.	Write short notes on the layout of distribution system.				 				   (8)

22.	a.	What is screening? Explain the various types of screens. 							    (8)

b.	Write short notes on the disposal of rakings, gritty materials and sludge solids.		   (7)

(OR)

23.	a.	Design a suitable rectangular sedimentation tank (provided with mechanical cleaning

equipment) for treating the sewage from a city provided with an assured public water supply system, with a maximum daily demand of 15 million litres per day. Assume suitable data.				   																    (10)

b.	Write short notes on short circuiting in the sedimentation tank.						    (5)

24.	Explain the Activated sludge process in detail with its relative advantage and disadvantages.

(OR)

25.	a.	Compare conventional trickling filter with High rate trickling filter.			            (10)	b.	Write short notes on low cost water treatments.					                                   (5)