Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	SURVEYING												Time : 3 hours

Subject Code:	CE280				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the principle of surveying

2.	What are bench marks

3.	What is Gales table

4.	What do the term face left mean

5.	Why is an anallatic lens provided in a tacheometer

6.	What is meant by orientation in plane table survey

7.	Name the various methods of setting out simple curve by chain and tape

8.	What is meant by vertical curve

9.	Classify triangulation system

10.	Define signal

PART – B  (5 x 3 = 15 MARKS)

11.	Compare height and collimation and rise and fall method used for calculation of reduced level

12.	Describe how temporary adjustment is done for theodolite

13.	Draw a neat diagram of subtense bar and state its use

14.	What are the functions of transition curve

15.	Name the corrections to be applied for base line measurement and state the formulas used

PART – C  (5 x 15 = 75 MARKS)

16.	The following consecutive readings were taken with a leveling instrument at intervals of 20m, 2.375,1.730, 0.615, 3.450, 2.835, 2.070, 1.835, 0.985, 0.435, 1.630,2.255, and 3.630m.The instrument was shifted after fourth and eighth reading. The last reading was taken on a B.M. of R.L. 110.200m. Find R.L. of all points.

(OR)

17.	a.	What are the various methods of contouring. Describe any two methods.            		    (10)

b.	State the Trapezoidal and Simpson’s rule.                           						     (5 )

18.	a.	Describe the process of measurement of horizontal angle by repetition and reiteration

method.  																	   (10)

b.	What should be the relation between fundamental lines of theodolite 				     (5)

(OR)

19.	The following records are obtained in a traverse survey where the length and bearing of the last  line were not recorded.

| Line | Length (m) | Bearing |
| --- | --- | --- |
| AB | 75.5 | 30O24I |
| BC | 180.5 | 110O36I |
| CD | 60.25 | 210O30I |
| DA | ----- | ----- |

Calculate the length and bearing of line DA									[P.T.O]

20.	The following observations were made using a tacheometer fitted with an anallatic lens, multiplying constant being 100.

| Instrument Station | Staff Station | WCB | Vertical angle | Stadia hair readings |
| --- | --- | --- | --- | --- |
| O | A | 30O30I | 4O30I | 1.155,1.755,2.355 |
| O | B | 75O30I | 10O15I | 1.250,2.000,2.750 |

Calculate the gradient of the line AB. Assume R.L. of instrument axis as 151.550 m.

(OR)

21.	a.	Describe how the three point problem can be solved.								 (9)

b.	What are the advantages and disadvantages of plane table survey?        				 (6)

22.	Explain the various surveys to be conducted for aligning a new highway project

(OR)

23. Calculate the ordinates at 7.5m intervals for a circular curve, given that the length of long chord is

60 m and radius 180m.

24.	The triangulation stations A and B 50 KM apart have elevations 243 m and 258 m respectively. The intervening ground may be assumed to have a uniform elevation of 216m. Find the minimum height of the signal required at B so that the line of sight may not pass nearer the ground than 2.4m.

(OR)

25.	a.	Write down the formula used for getting level difference between two points by single and

reciprocal observations.                                                             						     (8 )

b.	Explain how will you extend a base line                                             					(7)