Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	APPLIED ELASTICITY & PLASTICITY			              Time : 3 hours

Subject Code: 	CE302			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

Compulsory:

1.	Derive the expression for the bending of a beam with uniform load.

2.	a.	Derive the Differential equations of equilibrium for 2D problems.		(12)

b.	Derive the biharmonic equation in Cartesian co ordinate system		(8)

(OR)

3.	The stress components at a point are given by the following array:

105658106106

MPa

Calculate the principle stresses and principal planes.

4.	For a thin hollow disc of external radius “b” and internal radius “a” and for solid disc of radius “b” find the maximum stress in rotating disc.

(OR

5.	Derive the equation for pure bending of a curved bar.

6.	a.	Describe membrane analogy and derive the expressions of angle of twist, shear stress and

hence maximum shear stress induced in a circular section due to a twisting moment. Use prandtl’s stress function approach.												(12)

b.	A hallow Aluminium tube of rectangular cross section as  shown in Fig is subjected to torque of 56.5kN.m along its longitudinal axis. Determine the shearing stress and the angle of twist. Assume G =28Gpa													(8)

t2=0.006m

t1=0.012m

t3=0.01m

0.25m

t1=0.012m

0.5m

(OR)

7.	Derive the expression for St. Venants Warping Stress function.

[P.T.O]

8.	a.	Obtain the expression for Bending of beams of elastic perfectly plastic material for 	(12)

i.	Incipient Yielding.	ii.	Elastic plastic Yielding	iii.	Fully plastic Yielding

b.	A rectangular beam of width 4 cm and depth 6cm is 3m long and carries a concentrated load

of 2kN at mid span. If the stress strain curve for the beam material is given by σ=700 ∈0.25 Determine the maximum stress induced in the beam								(8)

(OR)

9.	Derive the equation for Tresca’s and Von Mises yield criteria.