Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FINITE ELEMENT METHODS IN ENGINEERING	              Time : 3 hours

Subject Code: 	CE304			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a. 	Using two point Gaussian quadrature function, evaluate the integral

I=y=c=4y=d=6y=a= −2y=b=2(1−x)2(4−y)2dxdy                                                       			(14)

b. 	Evaluate I=13dxx  using Gaussian three point formula.                               			(6)

2. 	a.  	For the two-dimensional loaded plate shown in the figure 1, determine the displacements of

nodes 1 and 2 and the element stresses using plane stress conditions. Take 1lb = 4.5 Newton, 1 inch = 25.4 mm, 1 psi = 6.89x10-3 N/mm2.                                                                  	(16)

Figure – 1

b.   State the Reissner’s Principle for Hybrid models.                                         				(4)

(OR)

3.	a. 	Find the deflection of a simply supported beam containing UDL of intensity “q” per metre

run over the entire span by Rayleigh-Ritz method and compare it with the

exact solution. 																 (16)

b. 	Draw the Pascal’s Triangle for a three-dimensional element.                         			 (4)

4.	a. 	Evaluate the shape functions N1, N2 and N3 at the interior point P for the triangular

element shown in figure 2.                                                               					(6)

[P.T.O]

Figure – 2

b. 		State the three main theorems of isoparametric elements.                              		(6)

c. 		Derive the area co-ordinates in Cartesian coordinate system for a Higher Order Triangular Element shown in figure 3. Consider the figure to be divided into 3 areas A1,A2 and A3.                                                                                        			(8)

Figure – 3

(OR)

5.	a. 	Derive the shape functions for an Eight-Node quadrilateral belonging to serendipity

family.		                       					     								(12)

b.  	Compute the consistent load vector when self weight of ρ per unit volume is acting in a

constant strain triangle element (ρ is acting in negative y direction).                           (8)

6.	a. 	Discuss in detail the 4 different approaches used to generate shell elements.			(16)

b. 	Draw a typical shell element and various stress resultants acting on it.          			(4)

(OR)

7.   	a. 	Discuss the isoparametric general shell element.                                             			(18)

b.	Write the stiffness equation for general shell element.                                       		(2)

8.	Discuss in detail the features of any four FEA software packages.            			 	(4 x 5)

(OR)

9. For a non-linear spring setup shown in figure 4, the force deflection characteristics are given by

Spring 1   F1 = 598x1 + 6060 x12

Spring 2   F2 = 657x2 + 919 x22

Spring 3   F3 = 69x3 + 196 x32

Figure - 4

The loads are: P1 = -120  and P2 = 398. Find the deflection in spring 1, 2 and  3 by using Newton-Raphson Method