Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DESIGN OF FOUNDATION STRUCTURES			              Time : 3 hours

Subject Code: 	CE309			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

Any missing data may be assumed suitably.

Compulsory:

1.	Design a pile group to carry a load of 5000 kN. The soil exploration data reveal that the sub soil consists of deposit of soft clay extending to a great depth. The other properties of clay are compression index = 0.10, initial void ratio= 0.90, Saturated unit weight = 19kN/m3, unconfined compression strength = 40kPa. Proportion the pile group for a maximum permissible settlement of 50 mm.

2.	From fundamentals, derive an expression for depth of embedment of cantilever sheet pile wall in

granular coil and in cohesive soil.

(OR)

3.	The following data refer to a cellular cofferdam. Design

a.	Circular cellular coffer dam and

b.	Diaphragm type cellular cofferdam.

Unit Weight of fill = 19 kN/m3.

Submerged unit Weight of Fill = 10 kN/m3.

Angle of Internal friction of fill material = 340.

Coefficient of Friction between the fill and rock = 0.65

Coefficient of Friction between the fill and sheet pile = 0.45

Coefficient of interlock friction = 0.35

4.	List out different dynamic properties of soil. Describe any two test procedure to determine the   dynamic properties of soil in the field.

(OR)

5.	Design a foundation for an impact machine for the following data

Weight of Ram = 15 kN

Height of fall = 1100mm

Weight of anvil = 300 kN

Efficiency of fall of ram = 92%

Coefficient of restitution = 0.50

Steam pressure = 600 kPa

Area of piston = 0.15m2

Limiting frequency of anvil = 240 radiance/ second.

Natural Frequency of the combined system = 275 rad/sec.

6.	Design a wall type break water for the following data

Depth of water = 25m

Wind velocity 150 kmph

Fetch of wave = 200 km

(OR)

[P.T.O]

7.	Design a rubble mound type break water for the following data

Depth of water = 20m

Wind velocity 120 kmph

Fetch of wave = 220 km

8.	Write a detailed note on following

a.	Identification of expansive soils	b.	Determination of swell potential

c.	Determination of  swell pressure	d.	Suitable foundation for expansive soils

(OR)

9.	Design a reinforced earth retaining wall 6.0 m height retains cohesion less soil. Unit weight of soil is 18 kN/m3 and the angle of repose is 350. The allowable soil pressure is 180 kPa . The safe design strength of geogrid reinforcement is 18 kN/m. Assume μ and α as 0.55 and 0.95 respectively.