Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	STRUCTURAL DYNAMICS						              Time : 3 hours

Subject Code: 	CE328			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the different types of damping.                                                                         	(6)

b.	Derive the solution for SDOF with damped harmonic excitation?                              	(14)

2 	a.	State Hamilton’s principle and its applications?                                                             (6)

b.	A vibrating system consists of a weight of 50kN and a spring with   stiffness k=5 N/mm

is viscously damped so that the ratio of the two Consecutive amplitudes is 1.00 to 0.85

Determine the following 														(14)

i.	Natural frequency of undamped system 		ii.	Logarithmic decrement

iii.	Damping Ratio							iv.	Damped Natural Frequency.

(OR)

3.	a.	Describe in detail the following methods for evaluation of damping in SDOF system. (6)

i) Free vibration Decay method

ii) Resonant Amplification Method

iii) Half Power or Band width method

b.	A platform of weight 20kN is being supported by four equal columns which are clamped to the foundation as well as to the platform. Experimentally it has been determined that a static force of 5 kN applied horizontally to the platform produces a displacement of 0.25cm. It is estimated that damping in the structure is 5% of the critical damping. Determine the following i) Undamped natural frequency, ii) Absolute damping coefficient   ii)Logarithmic decrement .         												(14)

4.	A water tank is subjected to a dynamic load as shown in the fig Evaluate numerically the Duhamel Integral and find the response take Δt =0.02 sec

(OR)

[P.T.O]

5. Express the periodic function shown in the fig in the form of Fourier series

6. 	a.	Discuss in detail the finite element method applicable to vibration analysis             	(10)

b.	Determine the first three natural frequency and corresponding mode shapes of a simply supported beam having a cross section 250 mm wide and 60mm deep with a  span of 11 m. Assume the flexural stiffness of the beam EI= 1 x10 13 Nmm 2  and  weight of the beam

w= 25kN/m 3 ( neglect the shear distortion and rotary inertia)                                     	(10)

(OR)

7. 	a.	Define the following  i) Response spectrum    ii) Lumped mass and Consistent mass  	(6)

b.	A simply supported beam of constant cross section is executing simple harmonic

motion. Find the value of the different natural frequencies                                         	(14)

8  	a.	Write Notes on  i) Orthogonal   property of Mode shapes      ii) Modal Analysis

iii) Advantages of flexibility formulations over stiffness modulations  				(6)

b.	Analyse the frame by Holzer’s method for their frequencies and mode shapes.     	(14)

(OR)

9. 	a.																				(6)

i.	Distinguish between Wilson  θ method and new Mark method

ii.	What do you mean  by shear building approximation.

iii.	Give the step by step procedure for the solution of equilibrium equation in dynamic

analysis using Houbolt method.

b.	Determine the frequency and mode shapes for the following frame using Rayleigh’s

method                                                                    								(14)