Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ANALYSIS AND DESIGN OF PLATE AND SHELL STRUCTURES													              							Time : 3 hours

Subject Code: 	CE330			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Design of hyperbolic parabolic shell of the tilted umbrella type of length 60m, width of 15 m, depth of 2 m, thickness of 80 mm, live load of 1.0 kN/m2 of shell surface.

2.	Derive Levy and Navier’s solution of plates.

(OR)

3.	Derive differential equations of laterally loaded and thin rectangular plates.

4.	Design of a north light folded plate of span 20 m and breadth 6 m and live load of  1.0 kN/m of surface area

| Plate | Plate width, m | Horizontal projection, m | Inclination to horizontal | Angle between plate and next plate | Thickness, mm |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.6 | 0.1 | 800 | 800 | 90 |
| 2 | 1.2 | 1.2 | 00 | 40035’ | 100 |
| 3 | 3.6 | 2.8 | (3600-40035’) | (3600-40035’) | 90 |
| 4 | 1.2 | 1.2 | 00 | (3600-800) | 100 |
| 5 | 0.6 | 0.1 | 800 | --- | 90 |

(OR)

5.	a.	Write the assumptions of folded plates.

b.	Write short notes on

i)  	Plate and slab action		ii)  	Resolution of ridge loads

iii)  Edge shear		    		iv)  	Stress distribution.

6.	a.	Classify the shells and discuss in detail about them

b.	Discuss about structural behaviour of thin shells.

(OR)

7.	Derive membrane theory of the parabolic conoid type 1.

8.	Design the single short cylindrical shell without edge beams of span 12 m, radius of 25 m, thickness of 0.25 m, semi central angle of 37048’ and live load of 1.0kN/m.

(OR)

9.	Analysis of a long interior cylindrical shell with prestressed edge beams with  shell geometry of span 55 m, radius of 9 m, thickness of 0.075 m, semi central angle of 39.50, edge beam of depth 2.5 m and width 0.1 m and live load of 1.0kN/m.