Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	STABILITY OF STRUCTURES						              Time : 3 hours

Subject Code: 	CE332			         						                Maximum Marks: 100

Stability Functions Chart / Table is permitted

Assume any missing data suitably.

Answer ALL questions (5 x 20 = 100 Marks)

Compulsory:

1.	a.	Obtain the buckling load due to symmetrical buckling of cylinder under uniform

compression 																	(14)

b.	Differentiate between perfect and imperfect systems.								(3)

c.	Write a not on path switching.													(3)

2.	a.	Plot the load deflection curve for an ideal column and a real column and indicate the

differences between the two.													(6)

b.	Obtain the critical load of a column with one end fixed and the other free.			(14)

(OR)

3.	Determine the value of the critical load of a linged-linged column shown in Fig.1 by energy

method or by finite difference method.

I=I(1+2xl)

for     0<x<L/2

x

I

I

L

Fig.1

4.	a.	Why tangent modulus load, rather than double modulus load is close to experimental critical

load on columns? Explain shanley’s model to support your answer.					(10)

b.	An idealized I section of a material having bilinear stress strain characteristics (E upto yield 		and (5/6) beyond yield) forms the cross section of a pin ended column supporting an axial 		load p at the free end. Determine the critical load based on

i.	tangent modulus theory.	b.	Double modulus theory.					(10)

(OR)

[P.T.O]

5.	Find the critical load of the frame shown in Fig.2 which will buckle laterally.

P

5m

2.5m

C

A

B

I

2I

6m

1.5I

Fig.2

P

6.	a.	Explain torsional-flexural buckling with examples.								 (5)

b.	A cantilever beam of uniform cross-section and length ‘L’ is subjected to a lateral load at the end through the centroid of the section. Find the buckling load.					(15)

(OR)

7.	Obtain the differential equation for non-uniform torsion in thin walled sections. Hence obtain the buckling load expression for a symmetrical I section.

8.	Determine the critical load for a plate subjected to uniform axial compression Nx per unit length and show graphically how the stress-coefficient is affected by the aspect ratio of the plate. Assume the plate is simply supported on all its edges.

(OR)

9.	Write short notes on the following.

a.	Energy concepts in stability of plates.											(6)

b.	Finite Difference method for stability of plates.									(7)

c.	Post buckling behaviour of plates.												(7)