Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ADVANCED DESIGN OF REINFORCED CONCRETE STRUCTURES														              					Time : 3 hours

Subject Code: 	CE364			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Estimate the deflection of a cantilever beam of breadth 300 mm, total depth 600mm, span 4m subjected to a maximum bending moment due to characteristic dead and live loads of 210 KN-m of which 60% is due to permanent loads.

Assume tension steel is 1.17%, compression steel 0.418%, cover  to centre of steel is 37.5 mm, creep factor 1.6; shrinkage strain = 0.003, grade of concrete M20, grade of steel Fe415.

(OR)

2.	A T-beam is as shown in Fig 1. If the section is subjected to the following factored loads; bending moment of 215 KN-m shear of 150 KN and torsion of 105 KN-m. Assuming                 fck = 30 N/mm2 and fy = 415 N/mm2. Design the reinforcements according to IS 456. Cover to centre of steel is 50mm.

700

120

350

800

Fig 1

3.	Design the longitudinal steel for a braced column 500 x 400 mm bent in single curvature with  fck = 40 N/mm2 and fy = 415 N/mm2 and the following data : Effective length about both axes         = 6.5 m, unsupported length = 7m. Factored load = 2500 KN, factored moments on major axis are 250 KN-m at the top and 200 KN-m at the bottom. The factored moments about minor axis are 120 KN-m at top and 100 KN-m at the bottom.

(OR)

4.	Fig 2 shows an arrangement for a coal bunker. Design the bunker. Assume suitable data.

2.5 m sq.

3m

1m

2.75 m

Column 300mm x 300mm

1m

Fig 2

[P.T.O]

Compulsory:

5.	a.	Write the assumptions made in yield-line theory of slabs                            			( 4 )

b.	Investigate the ultimate load capacity of a 4m x 6m slab continuous on all edges if the yield moments are 16 KN-m/m and 20 KN-m/m  for the positive and negative moments respectively, they being uniform in each condition.                           			      (16)

6.	a.	What are the advantages and disadvantages of moment redistribution      			( 6 )

b.	A tee beam ABC is continuous over two spans of 8m each  and it carries  uniformly distributed factored load of 75 KN/m. Assuming fck = 25 N/mm2 and fy = 415 N/mm2 (with bilinear stress strain curve) check whether we can reduce the maximum moment  by 30% and redistribute to the spans. Width of flange 1000mm, width of web = 300mm, thickness of slab = 150 mm D = 820 mm d = 770mm are given.    (14)

(OR)

7.	A reinforced concrete frame shown in Fig 3 has beams of 300 x 700 mm and column 300 x 400

mm. The columns are 3.5 m in height. The characteristic dead and live loads are 40 KN/m and 60 KN/m on the beam for roof and floors. Determine the direct load and bending moments for  which the internal column has to be designed at each level, using the approximate method. Assume  there are three floors only.

6 m

4 m

Fig 3

8.	A reinforced  concrete grid floor is to be designed to cover a floor area of size 12m by 16m. The spacings of the ribs in mutually perpendicular directions being 2m c/c. live load 1.5 KN/m2. Adopt M20 grade concrete and tor steel. Analyse the grid floor for moments and shears. Design suitable reinforcement at critical sections.

(OR)

9.	Determine the thickness and reinforcements for a simply-supported transfer girder of length 5.25m loaded from two columns  at 1.75 m from each end with 3750 KN. The total depth of beam is 4.2m and width of support is 520mm. Assume grade 40 concrete and Fe415 steel.