Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ADVANCED DESIGN OF STEEL STRUCTURES	              Time : 3 hours

Subject Code: 	CE365			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

IS: 800 is permitted, 	IS : 875 is permitted, 	Assume any mission data suitably

Compulsory:

1.	A self supporting chimney is 80 m high and its diameter at top is 3m. Design the plate for the stack. Adopt wind pressure as per IS 875. The location of the plate is such that intensity of the wind pressure upto 30m height is 1.5 kN/m2

2.	a.	Write short notes on short and long beam columns								    (10)

b.	Design a column to carry an axial load of 1200 kN with an end moment of 90 kNm about its

major axis and 60kNm about its minor axis. The effective length of the column in 5m. (10)

(OR)

3.	a.	Write a note on elastic lateral torsional buckling.									    (10)

b.	Design a beam of 6m length inclined at an angle of 240 with its minor axis to carry a load of 9kN/m.																		     (10)

4.	Design a gantry girder for the following data

Crane load lifting capacity  -	200kN		Weight of crane excluding trolley – 150 kN

Weight of trolly – 75 kN					Minimum approach of crane hook -	 1.2m

Distance between centers of gantry girders – 20m.

Distance between centers of crane wheels  - 3m

Span of gantry girder – 6m 	Weight of rail section – 0.3 k/N    Height of rail -75 mm.

(OR)

5.	The span of the knee roof truss used over an industrial building of 30m ling is 18m. The spacing of the roof truss is 5m. The pitch of the roof is 1 in 5. The GI sheets are used for roof covering. The basic wind pressure is 1.5 kN/m2. the height of eaves above ground level is 10m. Propose suitable type of roof truss. Design the gable rafter and gable column.

6.	a.	Explain the bracing of the multi storied frames.									    (10)

b.	How will you design the members of a multi storied frames for the combined vertical and horizontal loading.															   (10)

(OR)

7.	Using portal method analyse the frame shown in figure 1.

100 KN

200 KN

4m

4m

3 m

4 m

3 m

Fig.1

8.	Design the continuous beam shown in Figure 2.

30kN/m

60kN/m

40 kN/m

4 m

6 m

4m

Fig.2

(OR)

9.	Find the shape factor of the sections shown in figure 3 and 4. Also find the plastic moment of resistance if the yield stress in 240 N/mm2

20 mm

20 mm

20 mm

4 cm

3 cm

26 cm

4 cm

12 cm

120 mm

Fig. 3

Fig. 4