Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	PRINCIPLES OF CHEMICAL ENGINEERING				Time : 3 hours

Subject Code:	CL202			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The number of moles of solute present in one litre of the solution if defined as ______________

2.	The unit of enthalpy is ________________

3.	At critical point in the water-ice-steam, the degree of freedom is ____________

4.	Operation used in industry for concentration of week liquor to produce thick liquor is

a)Drying	b) Crystallization		c) Evaporation

5.	The latent heat of sublimation is the sum of _____________ and _______________ at a given temperature.

6.	If the process condition in a power varies with time at any given point it is known as ________________________

7.	When the flow of a fluid in circular pipe is laminar, the friction factor depends only on the ____________________

8.	Generally vapours are ________________ and gases are _____________________

9.	Rotary pumps deliver varying quantity of fluid per revolution (True / False)

10.	The pressure drop length of a packed column can be calculated by Ergan equation 
(True / False)

PART – B  (5 x 3 = 15 MARKS)

11.	What is the mass flow rate of 300 m3/min of methane at 250OC and 730 mm Hg pressure?

12.	Distinguish between conversion and yield.

13.	What is first and second law of thermodynamics?

14.	Explain about Fanning friction factor.

15.	Explain the performance characteristics of a pump working with centrifugal force.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the mathematical procedure for Experimental curve fitting & Numerical

integration.																(12)

b.	A compound having molecular weight of 123 analyses						(3)

| Element | Mass percent |
| --- | --- |
| Carbon | 58.536 |
| Hydrogen | 4.065 |
| Nitrogen | 11.382 |
| Oxygen | 26.016 |

Find the molecular formula of the compound?

(OR)

[P.T.O]

17.	a.	Explain about Vander Waal`s equation of state.								(5)

b.	A solution of sodium chloride in water contains 230 grams of NaCl per liter at 20oC. The density of the solution is 1.148 grams per cubic centimeter. Calculate the following 	(10)

i.	Composition in weight percent

ii.	Volumetric percent of water

iii.	Composition in mole percent

iv.	Molality

v.	lb of NaCl per lb of H2O

18.	a.	An aqueous solution of a high molecular weight solute is concentrated in an evaporator. The

feed is 200 m3/day with 4 (w/w) % solute. Density of the feed is 1020 kg/m3. The concentrated solution contains 30 (w/w) % solute. Calculate the following:		(12)

i.	Calculate the feed entering in kg / day

ii.	Calculate the mass of solute in the feed in kg / day

iii.	Calculate the mass of water in the feed in kg / day

iv.	How much of water is leaving the product?

v.	How much of water has been evaporated?

iv.	If the molecular weight of the solute is 204, what is its mole% in the feed and in the

product?

b.	Paper is passed continuously through tunnel drier. The entering paper contains 10% water

on dry basis and the paper leaving the drier contains 2% water on dry basis. How many kg/hr. of water is evaporated if 1000 kg/hr. paper enters the drier.				(3)

(OR)

19.	With a block diagram write the general overall & component material balance for

a) Evaporator	b) Extractor	c) Distillation	   d) Dryer	e) Filtration

20.	Derive and discuss in detail the mechanical energy equation with its corrected form, for fluid

friction.

(OR)

21.	Explain about the following

a.	Heat capacity of liquid mixtures		b.	Latent heat

c.	Adiabatic reactions					d.	Sensible heat changes in liquids

e.	Energy balances.

22.	Derive Ergan equation for fluid flows through packed bed.

(OR)

23.	Derive Bernoulli`s equation for the steady flow of an incompressible fluid, discuss the correction factors to be incorporated in the equation for practical applications.	(15)

24.	With a neat sketch explain the construction, performance & working principle of centrifugal pump.

(OR)

25.	Write short notes on:

a.	NPSH & Cavitation			b.	Priming		c.	Positive displacement compressor.