Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	UNIT OPERATIONS								               Time : 3 hours

Subject Code: 	CL204			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Propellers are used for fluids viscosity below about     _________.

2. 	Laminar flow mixers are used to mix _________ viscous mixtures.

3. 	A device that separates the solid into two fractions is called _______.

4. 	In a rotary-drum filter, the filter time is _________ than the total cycle time.

5. 	Rate of transfer process = _________.

6. 	Unsteady steady heat conduction occurs when _________.

7. 	_______ number is unimportant in forced convection.

8. 	The emissivity of a black body is _________.

9. 	Boiling point elevation of NaOH solution is almost independent of  ________.

10. 	Circulation pump is located below the evaporator to create ________.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the purposes of agitation?

12. 	Write a note on filter media.

13. 	Write down the significance of Biot number.

14. 	Define emissivity.

15. 	Define prandtle number and its significance.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	A disk turbine with 6 blades is installed centrally in a vertically baffled tank 1.5 m in dia.

The turbine is 0.65 m in dia and is positioned 0.65 m above the bottom of the tank. The turbine blades are 134 mm wide. The turbine impeller turns at 90 rpm. The tank is filled with a solution having a viscosity of 120 pa.sec and density of 1120 kg/m3. What power in kW will be required to drive the impeller?										(9)

b.	Derive an expression for power consumption in an agitation system for Newtonian fluids.

Draw the suitable correlation chart used.										(6)

(OR)

17. 	a.	Describe the different types of agitation equipments used for liquid agitation system. 	(9)

b.	Derive an expression for agitator scale up to obtain power per unit volume is constant (6)

18. 	Explain the basic theory of filtration and derive expression for constant-pressure filtration to obtain filter medium and specific cake resistance.

(OR)

[P.T.O]

19. 	a.	Using the filter press with an area of 0.0929 m2 performed constant-pressure filtration at

35.5 kpa of 13.9 wt % CaCO3 solid in water slurry at 300K. The mass ratio of wet cake to dry cakes was 1.59. The dry cake density was 1017 kg/m3. The data obtained are as follows, where W = kg filtrate and t = time in second.

| Weight | 0.91 | 1.81 | 2.72 | 3.63 | 4.54 | 5.44 | 6.35 | 7.26 | 8.16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time | 24 | 71 | 146 | 244 | 372 | 524 | 690 | 888 | 1188 |

i.	Calculate the specific cake resistance and filter medium resistance.				(10)

ii.	Explain the various stages in batch sedimentation with neat sketch.				(5)

20. 	a.	A thick walled tube of stainless steel having thermal conductivity 21.63 W /m K with

dimension of 0.0254m inner diameter and 0.0508m outer diameter is covered with 0.0254m layer of asbestos insulation of thermal conductivity 0.2423 W /m K. The inside wall temperature of the pipe is 811 K and the outer surface of the insulation is at 310 K for a 0.305m length pipe. Calculate the heat loss and also the temperature at the interface between the metal and the insulation.	         											(10)

b.	Derive an expression for combined conduction, convection and overall heat transfer

coefficient for a flat wall.														(5)

(OR)

21. 	a.	Derive from the fundamental the temperature distribution equation for a rectangular fin.

Assume the fin is very long.													(10)

b.	Derive an expression for determining the rate of heat transfer for conduction through a thick

walled hollow cylinder.	   													(5)

22. 	a.	Water flows through a tube of 5 cm inner diameter with velocity 1.2 m/sec. It is heated from

20°C to 80°C using steam at 160°C on the outer surface of the tube. Find heat transfer coefficient and length of tube required for transferring the above amount of heat. Neglect the tube wall resistance and outer surface film resistance. The properties of water are as follows. Density = 987 kg/m3, Cp = 4182 J/kg.K, Viscosity = 700 x 10-6 kg/m.sec and                                K= 0.643 w/m.k. Use suitable empirical correlation.   								(10)

b.	Explain the concept of drop wise and film type condensation. 						(5)

(OR)

23. 	a.	Explain how dimensional analysis is used in convection phenomenon where heat transfer

coefficient is found to be dependent on diameter, density, velocity, specific heat, thermal conductivity and viscosity. 													(10)

b.	Discuss the heat transfer coefficient for laminar and turbulent flow inside a pipe.	       	(5)

24. 	a.	Water is flowing at the rate1.5 kg/sec in a 1-2 shell and tube exchanger and is heated from

25 to 70°C by an oil having heat capacity of 1.95 kJ/kg K. The oil enters at 100°C and leaves at 65°C. Calculate the mass flow rate of oil and the area required for the heat exchange if the overall H.T coefficient is 415 W/m² K. 	  						(10)

b.	Derive mass and enthalpy balance for a multiple effect evaporator. 	         			(5)

(OR)

25. 	a.	Find the heating area required for concentrating a solution of 5000 kg/hr of 10% NaOH

entering at 30°C to a concentration of 50% NaOH evaporation is carried out in a single effect and the overall heat transfer coefficient is 2500 W/m² K. The evaporator is operated at 245 mmHg. Find the economy of the evaporator. Boiling point of water at 254 mmHg is 70°C, Boiling point of 50% NaOH is 110°C, Steam temperature is 150°C, Latent heat of steam is 510 kJ/kg, Enthalpy of feed is 35 kJ/kg, Enthalpy of product is 35 kJ/kg and enthalpy of vapor leaving is 600 kJ/ kg. 	   										(10)

b.	Explain the working principle of 2-4 shell and tube heat exchanger with neat diagram. (5)