Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	HEAT TRANSFER AND FLUID FLOW				               Time : 3 hours

Subject Code: 	CL209			         						                Maximum Marks: 100

Use of Heat Transfer Data Book Permitted

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are non-Newtonian fluids?

2.	Higher the surface tension higher will be the pressure inside a bubble.  (True or false)

3.	State the function of a nozzle.

4.	Define coefficient of discharge.

5.	Write the significance of Biot  number.

6.	What is meant by R value of insulating materials?

7.	Write the difference between thermal and hydrodynamic boundary layers.

8.	How is turbulent flow different from laminar flow?

9.	The ratio of pressure force to inertia force is called ___________

10.	Geometric similarity will automatically lead to kinematic similarity. True or false?

PART – B  (5 x 3 = 15 MARKS)

11.	State Pascal's law. State its significance.

12.	Explain how venturi meter can be used for determination of flow rate.

13.	Write the general three dimensional heat conduction equation in Cartesian form. Reduce it to Euler's equation.

14.	What is meant by mixing cup temperature? Write its general expression.

15.	State Buckingham Pi theorem and give its limitations.

PART – C  (5 x 15 = 75 MARKS)

16.	Two pipes of 0.35 m and 0.25 m diameter and length 2000 m and 1500 m with f values of 0.021 and 0.018 are connected in series to carry water from a reservoir to a supply system, the head available being 8 m. Determine flow rate neglecting minor losses.

(OR)

17.	Determine the rate of flow of water through a pipe of diameter 20 cm and length when one end of the pipe is connected to a tank and the other end is open to atmosphere. The pipe is horizontal and the height of water in the tank is 4 m above the centre of the pipe. Take f = 0.009 and ignore all minor losses.

18.	Derive an expression for the power transmitted through a nozzle and state the condition for maximum power transmitted.

(OR)

[P.T.O]

19.	Water flows through a triangular right angled weir first and then over a rectangular weir of 1 m width. The discharge coefficients of the triangular and rectangular weirs are 0.6 and 0.7 respectively. If the depth of water over the triangular weir is 36 cm, find the depth of water over the rectangular weir.

20.	Derive an expression for the temperature history of a solid subjected to transient heat transfer with its internal conduction resistance negligible. Express your equation in terms of dimensionless parameters.

(OR)

21.	Water flows at 50oC inside a 2.5 cm inside diameter tube such that hi = 3500 W/ m2K . The tube has a wall thickness of 0.8 mm and a thermal conductivity of 19 W/m-K. The outside of the tube loses heat by free convection with ho = 7.6 W/ m2-K. Calculate the overall heat transfer coefficient and heat loss per unit length to surroundings at 20o C.

22.	Engine oil at 20o C flows over a 20-cm square plate at a velocity of 1.2 m/s. The plate is heated to a uniform temperature of 60o C. Calculate the heat lost by the plate.

(OR)

23.	Air at 2 atm and 200 deg.C is heated as it flows at a velocity of 12 m/s through a tube of diameter 3 cm. A constant heat flux condition is maintained at the wall and the tube wall temperature is 20 deg.C above the air temperature all through the tube. Calculate the heat transfer per unit length and the increase in bulk temperature of air over 4m length of the tube.

24.	Using a specific example explain how dimensional analysis can be made using Rayleigh's method.

(OR)

25.	Explain in detail the principles of model analysis and testing.