Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	UNIT OPERATIONS IN FOOD PROCESSING – II 			Time: 3 hours

Subject Code:	CL210			       						               	 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Explain Fick’s law of diffusion.

2.	Indicate few unit operations are necessary to extend the shelf life of product

3.	Write the formula to calculate the terminal velocity.

4.	Compare and Contrast Sedimentation and Centrifugation.

5.	Rotary filter is also called as  …………

6.	Show the schematic diagram of Basket Centrifuge.

7.	 …………..  used for desalination process.

8.	List the different types of  pumps used in food industries

9.	Indicate the advantages of extrusion cooking.

10.	Give the applications of Twin screw extruders.

PART – B  (5 x 3 = 15 MARKS)

11.	Name some adsorbents and state its physical properties .

12.	Differentiate Thickeners and Clarifiers.

13.	Calculate terminal velocity if the centrifuge has a disc diameter of 500 mm, contains 144 discs with a rotational speed of 4250 rpm and volumetric flow rate is 0.1 m3s-1  (Hint ∑=98*103 m2).

14.	How does reverse osmosis work?

15.	What is the principle of Multihearth roasters.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	How the penetration and surface renewal theory used to determine the mass transfer

coefficients.

b.	Explain the types of solid diffusion.

(OR)

17.	a.	How the diffusion and migration process  are necessary to extend the shelf life of product.

b.	Derive the flux equation for the following cases

i.	Steady state Diffusion of gas A through a stagnant gas B

ii.	Equimolar  counter diffusion

18.	Broadly describe the working principle of different types of filters and clarifiers.

(OR)

19.	You are filtering a beer containing  6% citric acid on a continuous  rotary vacuum filter. The filter has an area of 18.1 m2 , a negligible medium resistance , a cycle time of 75 sec and a pressure difference of close to 1 atm. The cake which forms has a washing efficiency of only 60%, but it is incompressible and permeable:

/2p = 86 sec/cm2

[P.T.O]

The cake retains 7% of filtrate leaving and should be washed until the cake contains only 10% citric acid originally entrained. Calculate the filtration time and washing time required to process 3000 litres /hr of beer.

20.	A bowl centrifuge is used to concentrate a suspension of Escherichia Coli prior to cell disruption. The bowl of this unit has an inside radius of 12.7 cm and a length of 73.0 cm. The speed of the bowl is 16,000 r/min and the volumetric capacity is 200 litres/hr. Under these conditions, this centrifuge works well. Calculate the settling velocity Vg for the cells. After disruption the diameter of debris is about One half of the original diameter and the viscosity is increased four times. Estimate the volumetric capacity of this same centrifuge operating under these new conditions.

(OR)

21.	a.	With a neat sketch, explain the following type of centrifuge and its applications

b.	Tubular Bowl Centrifuge  ii) Disc Nozzle Cenrifuge.

22.	a.	Discuss the models applicable to membrane separation processes.

b.	Outline the different configurations of membrane separation processes.

(OR)

23.	a.	Define 	i.	Nanofiltration	ii.	Ultrafiltration	iii.	Microfiltration.

b.	Define concentration polarization. How to avoid concentration polarization in membrane separation processes.

24.	Classify the Extruders and depict its working principle and its applications.

(OR)

25.	Why roasters are needed in coffee industries? Mention the different types of roasters.